# 2026 SPP Transmission Expansion Plan 
(STEP) Report

Subtitle

By SPP Engineering

Published on March 30, 2026

Version 1

# Revision History

| Date or version number | Author | Change Description | Comments |
| --- | --- | --- | --- |
| 3/30/2026 | SPP Staff | Submitted to MOPC | No changes resulting from MOPC |

Contents

Revision History	i

Executive Summary	1

Portfolio Summary	3

Project Summary	6

NTC and Upgrade Activity	9

Upgrades Completed	9

Upgrades Approved for Construction	20

Upgrades/NTCs Withdrawn	98

In-Service Date Delays	100

Study Overviews	101

Transmission Service Studies	101

Completed Aggregate Facility Studies	101

Attachment AQ Requests	101

Generator Interconnection Studies	101

Transmission Planning (ITP)	101

Balanced Portfolio	102

High-Priority Studies	102

Sponsored Upgrade Studies	102

Interregional Studies	102

ITP20 Upgrades	104

Appendix 1	108

# Executive Summary

| January 1, 2025-December 31, 2025 |  |  |
| --- | --- | --- |
| 155 completed upgrades | 749 issued upgrades | 12 withdrawn upgrades |
| $1.1B | $14.1B | $15.5M |

The SPP Transmission Expansion Plan (STEP) is a comprehensive listing of all transmission projects in SPP for the 20-year planning horizon. Projects in the current portfolio include all Network Upgrades for which construction activities are ongoing or those for which construction has completed but not all closeout requirements have been fulfilled in accordance with Section 13 in Business Practice 7060.1 Projects in the STEP portfolio include:

- Upgrades required to satisfy requests for transmission service (TS)

- Upgrades required to satisfy requests for generator interconnection (GI)

- Requests pursuant to Attachment AQ of this Tariff (AQ)2

- Approved projects from the Integrated Transmission Planning Assessment (ITP)3

- Approved high priority upgrades (HP)

- Endorsed Sponsored Upgrades (SP)

- Approved Interregional Projects (IR)

1 SPP OATT Business Practices

2 Pursuant to RR467, all Upgrades resulting from under attachment AQ are tracked independently of transmission service upgrades as of 6/1/2022.

3 2025 ITP upgrades for the Short-Term Reliability Projects were not issued NTCs in 2025 due to timing of Board Approval. NTCs were issued in the first quarter of 2026. 2025 ITP Competitive Upgrades were not issued NTCs in 2025 and are expected to follow the RFP schedule.

SPP actively monitors and supports the progress of transmission expansion projects, emphasizing the importance of maintaining accountability for areas such as regional grid reliability standards, firm transmission commitments, and SPP Open Access Transmission Tariff (tariff) cost recovery. SPP staff solicits quarterly feedback from the project owners to determine the progress of each approved transmission project. This report charts the progress of all SPP transmission projects approved by the SPP Board of Directors (Board) or through a Federal Energy Regulatory Commission (FERC)-filed service agreement under the tariff. A summary of the project portfolio is also included.

Stakeholder Action Requested. We invite stakeholders and all interested parties to submit any written comments on the projects included in the STEP via the SPP Request Management System (RMS). SPP solicits feedback on proposed solutions to transmission needs through stakeholder working groups and planning summits, as well as through meetings, teleconferences, web conferences, and via email or secure web-based workspaces. These meetings provide an open forum where all stakeholders have an opportunity to provide advice and recommendations to SPP to aid in the development of the STEP. In addition to these opportunities, we also invite stakeholders to provide SPP with any transmission needs they deem to be beneficial to the transmission planning process through our website or RMS.

Figure 1: STEP Upgrades by Status ($ Millions)

Portfolio Summary

SPP assigns a project status to all network upgrades based on the projected in-service dates provided by the Designated Transmission Owner (DTO) relative to the Need Date determined for the project. Project status definitions are provided below:

- Closed Out: Construction complete and in-service; all close-out requirements fulfilled. Upgrades are reported as Closed Out for one quarter and then removed from the active portfolio

- Complete: Construction complete and in-service

- Delay – Mitigation: Projected In-Service Date beyond Need Date; interim mitigation provided or project may change but time permits the implementation of project

- Delay – Mitigation Window: Behind schedule, interim mitigation pending or project may change but time permits the implementation of project

- Identified: Upgrade identified and included in Board-approved study results

- In Service: Upgrade is operational, but not all construction activities are complete

- Notification to Construct with Conditions (NTC-C) Project Estimate Window: Within the NTC-C Project Estimate (CPE) window

- Notification to Construct (NTC) Commitment Window: NTC/NTC-C issued, still within the 90-day written commitment to construct window and no commitment received

- On Schedule < 4: On Schedule within four-year horizon

- On Schedule > 4: On Schedule beyond four-year horizon

- Re-evaluation: Project active; pending re-evaluation

- RFP Issued: Responses for competitive upgrades are being submitted and/or reviewed

- Suspended: NTC/GIA suspended

Figure 2: STEP Upgrades by Type ($ Millions)

Figure 3: Upgrade Summary by Type4

4 Mileage not available for all upgrade categories.

# Project Summary

In adherence to the tariff and SPP Business Practice 7060 (BP 7060), SPP issues NTCs to DTOs to begin work on network upgrades that have been approved or endorsed by the SPP Board to meet the construction needs of the STEP, tariff, or RTO.

Figure 4 below shows the estimated cost by in-service year for all network upgrades resulting from studies identified in Attachment O of the tariff.

| Source Study | Complete | Closed Out | Delay | In Service | Commitment Window | On Schedule | Re- evaluation | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2006 STEP | $0 | $189 | $0 | $0 | $0 | $0 | $0 | $189 |
| 2007 STEP | $0 | $494 | $0 | $0 | $0 | $0 | $0 | $494 |
| 2008 STEP | $0 | $413 | $0 | $0 | $0 | $0 | $0 | $413 |
| 2009 STEP | $0 | $550 | $0 | $0 | $0 | $0 | $0 | $550 |
| 2010 STEP | $0 | $116 | $0 | $0 | $0 | $0 | $0 | $116 |
| 2012 ITP10 | $0 | $365 | $670 | $0 | $0 | $0 | $0 | $1,035 |
| 2012 ITPNT | $0 | $185 | $0 | $0 | $0 | $0 | $0 | $185 |
| 2013 ITPNT | $0 | $489 | $0 | $0 | $0 | $0 | $0 | $489 |
| 2014 ITPNT | $0 | $439 | $0 | $0 | $0 | $0 | $0 | $439 |
| 2015 ITP10 | $0 | $44 | $0 | $0 | $0 | $0 | $0 | $44 |
| 2015 ITPNT | $0 | $196 | $0 | $0 | $0 | $0 | $0 | $196 |
| 2016 ITPNT | $25 | $413 | $4 | $0 | $0 | $0 | $2 | $444 |
| 2017 ITP10 | $0 | $11 | $0 | $0 | $0 | $0 | $0 | $11 |
| 2017 ITPNT | $0 | $52 | $0 | $0 | $0 | $0 | $0 | $52 |
| 2018 ITPNT | $0 | $55 | $0 | $0 | $0 | $0 | $0 | $55 |
| 2019 ITP | $279 | $55 | $51 | $0 | $0 | $0 | $0 | $385 |
| 2020 ITP | $221 | $72 | $24 | $0 | $0 | $0 | $0 | $317 |
| 2021 ITP | $244 | $43 | $832 | $0 | $0 | $0 | $0 | $1,119 |
| 2022 ITP | $1 | $4 | $0 | $1 | $0 | $6 | $0 | $12 |
| 2023 ITP | $158 | $11 | $302 | $0 | $0 | $24 | $2 | $497 |
| 2024 ITP | $69 | $2 | $8,186 | $112 | $376 | $1,514 | $0 | $10,259 |
| 2024 JTIQ (DOE) | $0 | $0 | $593 | $0 | $0 | $0 | $0 | $593 |
| 2025 ITP | $0 | $0 | $0 | $0 | $3,247 | $0 | $0 | $3,247 |
| AG Studies | $0 | $964 | $0 | $8 | $0 | $163 | $0 | $1,135 |
| Balanced Portfolio | $0 | $823 | $0 | $0 | $0 | $0 | $0 | $823 |
| DPA Studies | $187 | $581 | $81 | $0 | $0 | $79 | $0 | $928 |
| GI Studies | $606 | $331 | $36 | $477 | $5 | $333 | $0 | $1,788 |
| HPILS | $0 | $569 | $11 | $0 | $0 | $0 | $0 | $580 |
| IS Integration | $0 | $272 | $0 | $0 | $0 | $0 | $0 | $272 |
| Priority Projects | $0 | $1,318 | $0 | $0 | $0 | $0 | $0 | $1,318 |
| Sponsored Upgrades | $0 | $44 | $0 | $0 | $0 | $0 | $0 | $44 |
| Grand Total | $1,789 | $9,099 | $10,791 | $598 | $3,627 | $2,119 | $4 | $28,027 |

Table 1: Upgrade Cost by Study and Status ($ Millions)

Table 1 data covers all projects issued an NTC or NTC-C, included in an active Generator Interconnection Agreement (GIA), or identified in the tariff as a base plan upgrade. This table includes costs for network upgrades no longer included in the active Project Tracking Portfolio.

Figure 4: Upgrade Cost by In-Service Year ($ Millions)

# NTC and Upgrade Activity

In adherence with the tariff and BP 7060, SPP issues NTCs to DTOs to begin work on network upgrades that have been approved or endorsed by the Board to meet the construction needs of the tariff or RTO. Figure 5 and Table 2 show completed upgrades. Figure 6 and Table 3 show upgrades issued NTCs. Figure 7 and Table 4 show upgrades where the NTC was withdrawn during the reporting period.

### Upgrades Completed

Figure 5: Completed Upgrades

| NTC ID | IN-SERVICE DATE | UID | OWNER | UPGRADE NAME | SOURCE STUDY | COST ESTIMATE |
| --- | --- | --- | --- | --- | --- | --- |
| 210540 | 11/20/2025 | 112504 | OGE | Sooner - Wekiwa 345 kV Terminal Upgrades (OGE) | 2019 ITP | 5,642,819 |
| 210544 | 11/20/2025 | 112502 | AEP | Sooner - Wekiwa 345 kV Terminal Upgrades (AEP) | 2019 ITP | 7,651,684 |
| 210545 | 11/21/2025 | 112377 | WFEC | Anadarko 138 kV Breakers | 2019 ITP | 1,190,980 |
| 210545 | 12/31/2024 | 112378 | WFEC | Washita 69 kV Breaker | 2019 ITP | 370,000 |
| 210593 | 11/20/2025 | 112458 | TSOK | Sooner - Wekiwa 345 kV New Line | 2019 ITP | 144,559,958 |
| 210626 | 7/15/2025 | 122598 | NEET | Blackberry - Wolf Creek 345 kV | 2019 ITP | 87,298,154 |
| 220789 | 7/15/2025 | 112509 | EKC | Wolf Creek 345kV Terminal Equipment | 2019 ITP | 28,263,970 |
| 210574 | 3/3/2025 | 122725 | SPS | Allen - Quaker 115 kV Ckt 1 Rebuild | 2020 ITP | 3,533,803 |
| 210574 | 3/3/2025 | 122727 | SPS | Allen - Lubbock South 115 kV Ckt 1 Rebuild | 2020 ITP | 6,232,467 |
| 210574 | 5/8/2025 | 122876 | SPS | Deaf Smith #6 - Hereford 115 kV Ckt 1 Rebuild | 2020 ITP | 7,325,142 |
| 210574 | 2/7/2025 | 143167 | SPS | Cargill - Friona 115 kV Ckt 1 Rebuild | 2020 ITP | 2,172,456 |
| 210575 | 12/29/2025 | 122730 | AEP | South Shreveport - Wallace Lake 138 kV Ckt 1 Rebuild #2 | 2020 ITP | 20,575,323 |
| 210581 | 7/11/2025 | 122746 | BEPC | Watford City 230/115 kV Transformer Ckt 2 (KV2A | 2020 ITP | 173,041 |
| 210583 | 11/21/2025 | 122804 | EM | Northeast 161 kV Breakers | 2020 ITP | 1,827,550 |
| 210586 | 11/21/2025 | 122864 | WFEC | Anadarko 138 kV Breakers #2 | 2020 ITP | 405,000 |
| 210616 | 1/29/2025 | 122848 | OGE | Minco 345 kV Terminal Equipment | 2020 ITP | 5,170,350 |
| 210616 | 1/1/2025 | 122858 | OGE | Cimarron - Draper 345 kV Terminal Upgrades #2 | 2020 ITP | 647,852 |
| 210616 | 1/23/2025 | 143176 | OGE | Draper 345 kV Terminal Equipment | 2020 ITP | 6,264,468 |
| 210627 | 12/11/2025 | 122796 | AEP | Chisholm Substation 345 kV Terminal Upgrades | 2020 ITP | 8,289,302 |
| 210627 | 12/11/2025 | 143182 | AEP | Chisholm - Woodward - Border 345 kV Ckt 1 (AEP) | 2020 ITP | 12,238,460 |
| 210628 | 7/31/2025 | 122823 | OGE | Border - Woodward Tap 345 kV Substation | 2020 ITP | 23,239,179 |
| 210670 | 1/29/2025 | 133085 | NEET | Minco - Pleasant Valley 345 kV Ckt 1 | 2020 ITP | 38,537,430 |
| 210670 | 1/23/2025 | 133106 | NEET | Draper - Pleasant Valley 345 kV Ckt 2 | 2020 ITP | 16,516,041 |
| 210645 | 6/25/2025 | 122741 | NPPD | Scottsbluff - Victory Hill 115kV Ckt 2 | 2021 ITP | 17,809,299 |
| 210645 | 6/25/2025 | 143599 | NPPD | Scottsbluff 115 kV Ckt 2 Terminal Upgrades | 2021 ITP | 1,349,512 |
| 210645 | 6/1/2025 | 143600 | NPPD | Victory Hill 115 kV Ckt 2 Terminal Upgrades | 2021 ITP | 2,306,155 |
| 210648 | 4/17/2025 | 122734 | WAPA | Oahe 230 kV Terminal Upgrades (WAPA) | 2021 ITP | 770,086 |
| 210648 | 4/17/2025 | 143613 | WAPA | Sully Buttes - Whitlock 230 kV Line Clearance Increase | 2021 ITP | - |
| 210648 | 4/17/2025 | 143614 | WAPA | Whitlock - Glenham 230 kV Clearance Increase | 2021 ITP | 323,364 |
| 210656 | 6/30/2025 | 143806 | OGE | Marietta Tap (OKGE) - Marietta Switchyard (WFEC) 138 kV Ckt 1 New Line (OGE) | 2021 ITP | 892,961 |
| 210656 | 7/30/2025 | 143815 | OGE | Marietta 2 138 kV Voltage Conversion | 2021 ITP | 3,340,351 |
| 210656 | 9/19/2025 | 143816 | OGE | Rocky Point - Marietta 2 138 kV Ckt 1 Rebuild | 2021 ITP | 29,559,659 |
| 210656 | 3/15/2025 | 143817 | OGE | Rocky Point 69 kV Replace Relays | 2021 ITP | 230,807 |
| 210657 | 7/30/2025 | 144173 | WFEC | Marietta Switchyard 138 kV Ckt 1 Terminal Equipment | 2021 ITP | 704,000 |
| 210663 | 2/25/2025 | 144148 | MWE | E Newtown 115 kV New Statcom | 2021 ITP | 19,500,000 |
| 210663 | 2/25/2025 | 144202 | MWE | E Newtown 115 kV Substation | 2021 ITP | 1,500,000 |
| 210681 | 5/13/2025 | 156407 | SPS | Crossroads 345 kV ckts 1 & 2 Terminal Equipment | 2021 ITP | 22,249,517 |
| 210681 | 11/7/2025 | 156408 | SPS | Hobbs 345 kV ckts 1 & 2 Terminal Equipment #1 | 2021 ITP | 24,038,399 |
| 210681 | 4/25/2025 | 156409 | SPS | Hobbs 345 kV ckts 1 & 2 Terminal Equipment #2 | 2021 ITP | 17,368,687 |
| 210681 | 4/10/2025 | 156410 | SPS | Roadrunner 345 kV ckts 1 & 2 Terminal Equipment | 2021 ITP | 12,173,404 |
| 210710 | 4/1/2025 | 156289 | EM | Craig 345 kV Relay | 2022 ITP | 181,260 |
| 210715 | 12/31/2025 | 156282 | WAPA | Fargo 230 kV Terminal Upgrade | 2022 ITP | 243,120 |
| 220739 | 12/1/2025 | 157241 | OGE | Seminole 345/138 kV Transformer Ckt 3 (138 kV) | 2023 ITP | 9,281,667 |
| 220739 | 12/1/2025 | 157242 | OGE | Seminole 345/138 kV Transformer Ckt 3 (345 kV) | 2023 ITP | 5,613,374 |
| 220739 | 6/22/2025 | 157274 | OGE | Woodward Nitrogen 69 kV Terminal Upgrade | 2023 ITP | 800,000 |
| 220739 | 12/1/2025 | 157390 | OGE | Southgate - Westmoore Tap 138 kV Extend Line | 2023 ITP | 6,158,692 |
| 220739 | 12/1/2025 | 157392 | OGE | Westmoore 138 kV Terminal Upgrade #2 | 2023 ITP | 3,308,294 |
| 220739 | 12/15/2025 | 157393 | OGE | Pennsylvania 138 kV Terminal Upgrade | 2023 ITP | 200,000 |
| 220742 | 1/15/2025 | 157949 | NWE | Yankton Junction 115 kV Ckt 1 Terminal Upgrade | 2023 ITP | 410,121 |
| 220743 | 12/10/2025 | 157594 | NPPD | Gerald Gentleman Station - Ogallala 230 kV Line & Terminal Upgrades | 2023 ITP | 1,758,786 |
| 220743 | 12/10/2025 | 157595 | NPPD | Ogallala 230 kV Terminal Upgrade | 2023 ITP | 103,000 |
| 220746 | 9/26/2025 | 157255 | EM | Blue Valley 161 kV Breaker #1 | 2023 ITP | 535,352 |
| 220749 | 11/18/2025 | 157403 | EKC | Neosho 345 kV Terminal Upgrade | 2023 ITP | 1,127,466 |
| 220749 | 12/18/2025 | 158452 | EKC | Benton 345 kV Terminal Upgrade | 2023 ITP | 783,591 |
| 220749 | 12/18/2025 | 158453 | EKC | Wichita 345 kV Terminal Upgrade | 2023 ITP | 147,584 |
| 220758 | 5/30/2025 | 157382 | WFEC | Anadarko 138 kV Terminal Upgrade Ckt 1 | 2023 ITP | 24,000 |
| 220791 | 4/2/2025 | 158640 | EKC | Atlantic Substation 115 kV | 2023 ITP | 16,068,122 |
| 220791 | 4/2/2025 | 158641 | EKC | Atlantic Substation 345 kV | 2023 ITP | 25,407,790 |
| 220791 | 5/16/2025 | 158645 | EKC | Eudora Township - Atlantic 115 kV Tap | 2023 ITP | 6,903,534 |
| 220791 | 8/14/2025 | 158648 | EKC | Atlantic 345/115 kV Transformer (115 kV) | 2023 ITP | 18,659,127 |
| 220791 | 4/2/2025 | 159064 | EKC | Atlantic Terminal Equipment 345 kV Transformer (345 kV) | 2023 ITP | 724,630 |
| 220791 | 7/18/2025 | 170552 | EKC | Atlantic – Pacific #1 115 kV New Line | 2023 ITP | 13,362,585 |
| 220791 | 4/2/2025 | 170553 | EKC | Atlantic - Pacific #2 115 kV New Line | 2023 ITP | 6,072,068 |
| 220792 | 4/2/2025 | 158646 | EM | West Gardner - Craig 345 kV | 2023 ITP | 30,189,542 |
| 220812 | 10/31/2025 | 169664 | OGE | Tinker 138 kV Breaker 121 Replacement | 2024 ITP | 225,000 |
| 220812 | 10/31/2025 | 169665 | OGE | Tinker 138 kV Breaker 122 Replacement | 2024 ITP | 225,000 |
| 220815 | 4/17/2025 | 159533 | WAPA | Bismarck 115 kV Terminal Upgrade | 2024 ITP |  |
| 220817 | 5/15/2025 | 159258 | SPS | Roadrunner 345/115 kV Transformer Ckt 2 (115 kV) | 2024 ITP | 1,901,723 |
| 220817 | 5/15/2025 | 159259 | SPS | Roadrunner 345/115 kV Transformer Ckt 2 (345 kV) | 2024 ITP | 446,984 |
| 220817 | 3/3/2025 | 159391 | SPS | Frankford - Quaker 115 kV Ckt 1 Rebuild | 2024 ITP | 5,464,908 |
| 220817 | 6/23/2025 | 159392 | SPS | Frankford 115 kV Ckt 1 Terminal Upgrade | 2024 ITP | 55,612 |
| 220817 | 5/5/2025 | 159393 | SPS | Quaker 115 kV Ckt 1 Terminal Upgrade | 2024 ITP | 65,549 |
| 220904 | 5/15/2025 | 169675 | EREC | Mount Vernon - Mitchell 69 kV Ckt 1 New Line | 2024 ITP | 2,472,000 |
| 220904 | 7/1/2025 | 169676 | EREC | Mitchell 69 kV New Substation | 2024 ITP | 3,947,000 |
| 220904 | 12/12/2025 | 169678 | EREC | Letcher 115 kV Ckt 1 Terminal Upgrade | 2024 ITP | 1,936,000 |
| 220904 | 11/3/2025 | 169686 | EREC | Mitchell - Hanson County 69 kV Ckt 1 New Line | 2024 ITP | 4,993,000 |
| 220904 | 11/3/2025 | 169692 | EREC | Plano - Hanson County 115 kV Ckt 1 New Line | 2024 ITP | 8,017,000 |
| 220904 | 8/1/2025 | 169694 | EREC | Plano 115 kV New Motor Operated Switch (MOS) | 2024 ITP | 170,000 |
| 220904 | 12/12/2025 | 169695 | EREC | Letcher-Miner County 115 kV line extension | 2024 ITP | 1,527,000 |
| 220904 | 11/7/2025 | 169696 | EREC | Hanson County 115 kV New Substation | 2024 ITP | 2,046,000 |
| 220904 | 11/7/2025 | 169697 | EREC | Hanson County 69 kV New Substation | 2024 ITP | 5,441,000 |
| 220904 | 11/7/2025 | 169698 | EREC | Hanson County 69 kV Capacitor | 2024 ITP | 963,000 |
| 220904 | 11/7/2025 | 169699 | EREC | Hanson County 115/69 kV Transformer Ckt 1 (69 kV) | 2024 ITP | 3,075,000 |
| 220904 | 9/3/2025 | 169701 | EREC | Mount Vernon 69 kV Ckt 1 Terminal Upgrade 3 | 2024 ITP | 800,000 |
| 220904 | 9/19/2025 | 169937 | EREC | Lake County - Howard 115 kV Ckt 1 New Line | 2024 ITP | 8,762,000 |
| 220904 | 11/14/2025 | 169940 | EREC | Howard - Howard City 115 kV Ckt 1 New Line | 2024 ITP | 1,962,000 |
| 220904 | 11/14/2025 | 169951 | EREC | Howard City - Miner County 115 kV Ckt 1 New Line | 2024 ITP | 4,742,000 |
| 220904 | 11/7/2025 | 169953 | EREC | Miner County 115 kV Ckt 1 Terminal Upgrade #2 | 2024 ITP | 760,000 |
| 220904 | 8/1/2025 | 169972 | EREC | Lake County 115 kV New Substation | 2024 ITP | 3,342,000 |
| 220904 | 11/6/2025 | 169975 | EREC | Howard City 115 kV New Motor Operated Switch (MOS) | 2024 ITP | 165,000 |
| 220904 | 5/1/2025 | 169978 | EREC | Miner County 115 kV Ckt 1 Terminal Upgrade #1 | 2024 ITP | 760,000 |
| 220904 | 10/1/2025 | 169979 | EREC | Lake County 115/69 kV Transformer Ckt 1 (69 kV) | 2024 ITP | 3,344,000 |
| 220904 | 10/31/2025 | 170488 | EREC | Arlington 115 kV Breakers | 2024 ITP | 3,846,000 |
| 210562 | 10/30/2025 | 122602 | WAPA | Jamestown 230 kV Redundant Bus Tie | DISIS-2016-002 | 1,741,183 |
| 200477 | 12/23/2025 | 71963 | NPPD | Monolith 345 kV Substation | DPA-2016-December-703 | 24,319,368 |
| 200477 | 12/23/2025 | 71964 | NPPD | Monolith 345/115 kV Transformer #1 | DPA-2016-December-703 | 5,468,352 |
| 200477 | 12/23/2025 | 71965 | NPPD | Monolith 345/115 kV Transformer #2 | DPA-2016-December-703 | 5,468,352 |
| 200477 | 12/23/2025 | 71968 | NPPD | Sheldon 115 kV Terminal Upgrades | DPA-2016-December-703 | 5,928,037 |
| 210682 | 12/23/2025 | 71966 | NPPD | Monolith 115 kV Substation Upgrades | DPA-2016-December-703 | 18,617,823 |
| 210483 | 9/20/2025 | 72018 | WFEC | Calumet - Watonga SW 138 kV Ckt 1 Voltage Conversion | DPA-2017-August-767-774-776 | 8,400,000 |
| 210483 | 12/30/2025 | 72019 | WFEC | Calumet - Concho 138 kV Ckt 1 Voltage Conversion | DPA-2017-August-767-774-776 | 2,150,000 |
| 210483 | 10/30/2025 | 72021 | WFEC | Calumet 138 kV Substation | DPA-2017-August-767-774-776 | 6,580,002 |
| 210489 | 12/31/2024 | 82114 | WFEC | Blanchard - OU SW 138 kV Ckt 1 Voltage Conversion | DPA-2017-December-815 | 623,621 |
| 210617 | 7/31/2025 | 143253 | WFEC | Paoli - Lexington kV 138 kV | DPA-2020-May-1203 | 10,608,616 |
| 210617 | 12/31/2025 | 143254 | WFEC | Lexington - Lil Axe 138 kV | DPA-2020-May-1203 | 7,378,510 |
| 210617 | 7/30/2025 | 143255 | WFEC | Paoli 138 kV Terminal Upgrades | DPA-2020-May-1203 | 2,225,000 |
| 220727 | 10/31/2025 | 144206 | SPS | Twist 115 kV Substation | DPA-2021-Feb-1279-1280 | 8,194,459 |
| 220727 | 12/19/2025 | 144207 | SPS | RB-Exum - Hilmar Cheese Ckt 1, Twist 115 kV New Lines, North Side | DPA-2021-Feb-1279-1280 | 4,905,051 |
| 220727 | 10/15/2025 | 144208 | SPS | RB-Kemp - Moore County Ckt 1, Twist 115 kV New Lines, south side. | DPA-2021-Feb-1279-1280 | 4,573,961 |
| 210619 | 6/6/2025 | 143705 | NPPD | Kearney NE - Tower 115 kV Ckt 1 New Line | DPA-2021-February-1278 | 11,074,254 |
| 210619 | 6/6/2025 | 144283 | NPPD | Tower 115 kV Terminal Equipment | DPA-2021-February-1278 | 3,689,073 |
| 210619 | 6/6/2025 | 144284 | NPPD | Kearney 115 kV Terminal Equipment | DPA-2021-February-1278 | 5,182,896 |
| 210664 | 11/28/2025 | 144279 | OGE | Classen 138 kV Terminal Upgrades | DPA-2021-March-1296 | 600,000 |
| 210664 | 8/1/2025 | 144280 | OGE | Council - Mustang 138 kV Ckt 1 Reconductor | DPA-2021-March-1296 | 1,390,000 |
| 210664 | 11/28/2025 | 144281 | OGE | Classen - Douglas Tap 138 kV Reconductor | DPA-2021-March-1296 | 790,000 |
| 220766 | 2/13/2025 | 157302 | SPS | Roadrunner - Red Bluff 115 kV Reconductor | DPA-2022-March-1564 | 10,858,947 |
| 220766 | 5/22/2025 | 157304 | SPS | Livingston Ridge 115 kV Cap Bank | DPA-2022-March-1564 | 6,515,148 |
| 220766 | 4/4/2025 | 157308 | SPS | Malaga Bend 115 kV Cap Bank | DPA-2022-March-1564 | 4,180,163 |
| 220766 | 8/27/2025 | 157310 | SPS | North Loving 115 kV Cap Bank | DPA-2022-March-1564 | 3,100,327 |
| 220777 | 12/15/2025 | 157303 | SPS | Phantom 115 kV Cap Bank | DPA-2022-March-1564 | 6,465,378 |
|  | 4/11/2025 | 143691 | WAPA | Maryville to Creston 161kV Reconductor (GIA-61) | ASGI-2018-001 | 14,900,000 |
|  | 1/6/2025 | 144324 | SEPC | Russell 115kV GEN-2020-056 Interconnection (TOIF) (SUNC) | GEN-2020 -056 | 2,177,009 |
|  | 1/6/2025 | 144325 | SEPC | Russell 115kV GEN-2020-056 Interconnection (Non-Shared NU) (SUNC) | GEN-2020 -056 | 309,607 |
|  | 10/1/2025 | 159054 | EREC | Sully Buttes 115 kV Substation | DPA-2022-February-1539 | 3,600,000 |
|  | 10/1/2025 | 159055 | EREC | Sully Buttes 230/115 kV transformer | DPA-2022-February-1539 | 4,900,000 |
|  | 10/1/2025 | 159056 | EREC | Grey Goose Substation Conversion to 115 kV | DPA-2022-February-1539 | 1,100,000 |
|  | 10/1/2025 | 159057 | EREC | Hughes County 115 kV Bay Addition | DPA-2022-February-1539 | 600,000 |
|  | 2/28/2025 | 72032 | EKC | Wild Plains 345kV Switching Station GEN-2015-052 Interconnection (TOIF) | Generation Interconnection | 1,977,982 |
|  | 3/3/2025 | 72034 | OGE | Wild Plains 345kV Switching Station GEN-2015-052 Interconnection (NU - OGE) | Generation Interconnection | 20,000 |
|  | 9/23/2025 | 82128 | OGE | Tap Cleveland - Sooner 345 kV Substation GEN-2015-066 Interconnection (TOIF) | Generation Interconnection | 18,000,000 |
|  | 9/23/2025 | 82129 | OGE | Tap Cleveland - Sooner 345 kV Substation GEN-2015-066 Interconnection (NU) | Generation Interconnection | 18,000,000 |
|  | 3/31/2025 | 112341 | OGE | Blue River 138 kV Substation GEN-2016-102 Interconnection (TOIF) (OKGE) | Generation Interconnection | 410,000 |
|  | 3/31/2025 | 112342 | OGE | Blue River 138 kV Substation GEN-2016-102 Interconnection (Non-Shared NU) (OKGE) | Generation Interconnection | 2,995,000 |
|  | 7/29/2025 | 112343 | OGE | Arbuckle 138 kV Substation GEN-2016-126 Interconnection (TOIF) (OKGE) | Generation Interconnection | 410,000 |
|  | 7/29/2025 | 112344 | OGE | Arbuckle 138 kV Substation GEN-2016-126 Interconnection (Non-Shared NU) (OKGE) | Generation Interconnection | 3,590,000 |
|  | 3/15/2025 | 122635 | OGE | GEN-2016-119 Interconnection (TOIF) (OKGE) | Generation Interconnection | 1,099,871 |
|  | 8/18/2025 | 122662 | OGE | Woodring 345kV GEN-2016-128 Interconnection (TOIF) (OKGE) | Generation Interconnection | 15,000 |
|  | 12/31/2025 | 132992 | OGE | Canadian River - Muskogee - Seminole 345 kV GEN-2017-040 Interconnection (TOIF) (OKGE) | Generation Interconnection | 599,979 |
|  | 12/1/2025 | 132993 | OGE | Canadian River - Muskogee - Seminole 345 kV GEN-2017-040 Interconnection (Shared NU) (OKGE) | Generation Interconnection | 5,741,321 |
|  | 5/30/2025 | 133034 | OGE | Greenwood 138 kV Substation GEN-2017-071 Interconnection (TOIF) (OKGE) | Generation Interconnection | 515,000 |
|  | 5/30/2025 | 133036 | OGE | Greenwood 138 kV Substation GEN-2017-072 Interconnection (TOIF) (OKGE) | Generation Interconnection | 515,000 |
|  | 4/30/2025 | 133037 | OGE | Greenwood 138 kV Interconnection Expansion (DISIS-2017-001) | Generation Interconnection | 1,721,000 |
|  | 12/1/2025 | 133072 | OGE | Canadian River - Muskogee - Seminole 345 kV GEN-2017-092 Interconnection (TOIF) (OKGE) | Generation Interconnection | 599,979 |
|  | 12/30/2025 | 133073 | OGE | Canadian River - Muskogee - Seminole 345 kV GEN-2017-092 Interconnection (Non-Shared NU) (OKGE) | Generation Interconnection | 5,741,321 |
|  | 5/14/2025 | 143226 | BEPC | Broadland 345/230kV Transformer Replacement (DISIS-2017-001) | Generation Interconnection | 10,061,320 |
|  | 1/29/2025 | 143402 | OGE | Minco 345kV Interconnection Expansion (DISIS-2017-002) | Generation Interconnection | 4,627,433 |
|  | 8/18/2025 | 143424 | OGE | Woodring 345kV Interconnection Expansion (DISIS-2017-002) | Generation Interconnection | 1,790,448 |
|  | 8/18/2025 | 143425 | OGE | Woodring 345kV GEN-2017-164 Interconnection (TOIF) (OGE) | Generation Interconnection | 795,827 |
|  | 6/2/2025 | 143445 | OGE | Woodring 345kV GEN-2017-178 Interconnection (TOIF) (OGE) | Generation Interconnection | 795,827 |
|  | 11/13/2025 | 143540 | OGE | Brown 138kV GEN-2017-232 Interconnection (Non-Shared NU) (OGE) | Generation Interconnection | 3,299,998 |
|  | 11/13/2025 | 143541 | OGE | Brown 138kV GEN-2017-232 Interconnection (TOIF) (OGE) | Generation Interconnection | 3,300,000 |
| 220778 | 6/12/2025 | 159263 | BEPC | XFR - Judson 345/230 kV second (230 kV) | SPP-2023-AG2 | 8,300,000 |
| 210486 | 4/17/2025 | 72006 | WAPA | Bismarck 115 kV Terminal Upgrades | SUS-002 | 148,047 |
| 210661 | 10/1/2025 | 144328 | EREC | Carpenter - Willow Lake 69 kV | SUS-018 |  |
| 220723 | 5/22/2025 | 158447 | OPPD | S1281 - S1361 161 kV | SUS-038 | 3,111,647 |
| 155 Upgrades Complete |  |  |  |  |  | $1,098,557,248 |

Table 2: Upgrades Completed

### Upgrades Approved for Construction

Figure 6: Upgrades Issued NTCs

| NTC ID | NTC CREATION DATE | UID | OWNER | UPGRADE NAME | SOURCE STUDY | COST ESTIMATE OF NEW UPGRADES | COST ESTIMATE OF PREVIOUSLY APPROVED UPGRADES |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 220880 | 1/23/2025 | 159382 | EDE | N Reed Spring - S Reed Spring 161 kV Ckt 1 Rebuild | 2024 ITP | 2,353,884 | 0 |
| 220880 | 1/23/2025 | 159383 | EDE | N Reed Spring 161 kV Ckt 1 Terminal Upgrade | 2024 ITP | 786,451 | 0 |
| 220880 | 1/23/2025 | 159384 | EDE | S Reed Spring 161 kV Ckt 1 Terminal Upgrade | 2024 ITP | 126,095 | 0 |
| 220880 | 1/23/2025 | 159385 | EDE | Reed Spring - Branson Northwest 161 kV Ckt 1 Rebuild | 2024 ITP | 12,918,360 | 0 |
| 220880 | 1/23/2025 | 159387 | EDE | Branson Northwest 161 kV Ckt 1 Terminal Upgrade | 2024 ITP | 357,827 | 0 |
| 220880 | 1/23/2025 | 159438 | EDE | Branson North - Ozark Dam 161 kV Ckt 1 Rebuild | 2024 ITP | 11,489,688 | 0 |
| 220880 | 1/23/2025 | 159439 | EDE | Ozark Dam 161 kV Ckt 1 Terminal Upgrade | 2024 ITP | 249,323 | 0 |
| 220880 | 1/23/2025 | 159440 | EDE | Branson North 161 kV Ckt 1 Terminal Upgrade | 2024 ITP | 636,244 | 0 |
| 220880 | 1/23/2025 | 170401 | EDE | Branson Northwest - Branson North 161 kV Ckt 1 Rebuild | 2024 ITP | 1,347,741 | 0 |
| 220880 | 1/23/2025 | 170402 | EDE | Branson Northwest 161 kV Ckt 1 Terminal Upgrade #2 | 2024 ITP | 480,000 | 0 |
| 220880 | 1/23/2025 | 170403 | EDE | Branson North 161 kV Ckt 1 Terminal Upgrade #3 | 2024 ITP | 300,000 | 0 |
| 220884 | 1/10/2025 | 159327 | OGE | Colbert 138 kV Ckt 1 Terminal Upgrade | 2024 ITP | 451,006 | 0 |
| 220885 | 1/14/2025 | 170543 | EM | Hawthorn 345kV Terminal Equipment | 2024 JTIQ (DOE) | 1,688,000 | 0 |
| 220887 | 3/21/2025 | 169968 | BEPC | Holcomb - Sidney 345 kV Ckt 1 New Line | 2024 ITP | 977,728,865 | 0 |
| 220889 | 3/21/2025 | 170052 | SPS | Potter 765 kV Ckt 1 Terminal Upgrade | 2024 ITP | 2,137,638 | 0 |
| 220889 | 3/21/2025 | 170053 | SPS | Crossroads 765 kV Ckt 1 Terminal Upgrade | 2024 ITP | 2,137,638 | 0 |
| 220889 | 3/21/2025 | 170090 | SPS | Phantom 765 kV Ckt 1 Terminal Upgrade | 2024 ITP | 2,137,638 | 0 |
| 220889 | 3/21/2025 | 170104 | SPS | Crossroads 765 kV Ckt 1 Terminal Upgrade #1 | 2024 ITP | 2,137,638 | 0 |
| 220890 | 2/12/2025 | 159565 | LEA | Kingsmill - Grapevine 115 kV Ckt 1 New Line | 2024 ITP | 10,602,558 | 0 |
| 220890 | 2/12/2025 | 159566 | LEA | Kingsmill 115 kV Ckt 1 Terminal Upgrade | 2024 ITP | 3,425,298 | 0 |
| 220890 | 2/12/2025 | 159567 | LEA | Grapevine 115 kV Ckt 1 Terminal Upgrade | 2024 ITP | 13,695,573 | 0 |
| 220890 | 2/12/2025 | 170171 | LEA | Iron House 115 kV Ckt 1 Terminal Upgrade | 2024 ITP | 3,700,485 | 0 |
| 220893 | 2/20/2025 | 170542 | NPPD | Cooper - Moore 345 kV Line Tap | 2024 JTIQ (DOE) | 9,000,000 | 0 |
| 220893 | 2/20/2025 | 170569 | NPPD | Tekamah 345 kV Terminal Upgrade | 2024 JTIQ (DOE) | 2,500,000 | 0 |
| 220894 | 3/21/2025 | 170568 | TSGT | Sidney 345 kV Ckt 1 Terminal Upgrade (TSGT) | 2024 ITP | 7,322,518 | 0 |
| 220895 | 3/21/2025 | 170170 | LEA | Iron House - Texaco 115 kV Ckt 1 New Line | 2024 ITP | 19,900,000 | 0 |
| 220897 | 4/1/2025 | 170375 | SPS | Tyng 115 kV Capacitor Bank | DPA-2022-October-1665 | 3,268,143 | 0 |
| 220897 | 4/1/2025 | 170414 | SPS | Kingsmill 69 kV Line Terminal Upgrade | DPA-2022-October-1665 | 967,188 | 0 |
| 220898 | 4/3/2025 | 170376 | SPS | Frankford - Quaker 115 kV Reconductor | DPA-2023-January-1705 | 3,326,966 | 0 |
| 220899 | 4/3/2025 | 170406 | SPS | Sundown - Wolfforth 230 kV Rebuild | DPA-2023-February-1719 | 839,019 | 0 |
| 220899 | 4/3/2025 | 170366 | SPS | Ponderosa 28.8 MVAR cap bank | DPA-2023-February-1719 | 5,404,216 | 0 |
| 220899 | 4/3/2025 | 170367 | SPS | Battle Axe 115 kV Device | DPA-2023-February-1719 | 4,709,908 | 0 |
| 220899 | 4/3/2025 | 170368 | SPS | Kiowa 345 kV Capacitor Bank | DPA-2023-February-1719 | 19,077,307 | 0 |
| 220899 | 4/3/2025 | 170369 | SPS | Yoakum 230 kV Capacitor Bank | DPA-2023-February-1719 | 8,704,494 | 0 |
| 220899 | 4/3/2025 | 170370 | SPS | Pearle 115 kV Capacitor Bank | DPA-2023-February-1719 | 2,861,913 | 0 |
| 220901 | 4/24/2025 | 159345 | EKC | Farber - Sumner County No. 10 Belle Plain 138 kV Ckt 1 Rebuild | 2024 ITP | 19,222,237 | 0 |
| 220902 | 4/24/2025 | 159365 | OPPD | Sub 1209 - Sub 1250 161 kV Ckt 2 Rebuild | 2024 ITP | 28,933,550 | 0 |
| 220902 | 4/24/2025 | 159366 | OPPD | Sub 1209 161 kV Ckt 2 Terminal Upgrade | 2024 ITP | 373,879 | 0 |
| 220902 | 4/24/2025 | 159367 | OPPD | Sub 1250 161 kV Ckt 2 Terminal Upgrade | 2024 ITP | 141,570 | 0 |
| 220902 | 4/24/2025 | 170036 | OPPD | S3458 - S3740 345 kV Ckt 2 New Line | 2024 ITP | 97,406,244 | 0 |
| 220902 | 4/24/2025 | 170037 | OPPD | S3458 345 kV Ckt 2 Terminal Upgrade | 2024 ITP | 5206856 | 0 |
| 220902 | 4/24/2025 | 170038 | OPPD | S3740 345 kV Ckt 2 Terminal Upgrade | 2024 ITP | 3,386,900 | 0 |
| 220903 | 5/1/2025 | 169886 | EDE | Reeds Spring - Branson Northwest 161 kV Line Tap | 2024 ITP | 2,328,339 | 0 |
| 220903 | 5/1/2025 | 169887 | EDE | North Branson 161 kV New Substation | 2024 ITP | 34,201,800 | 0 |
| 220903 | 5/1/2025 | 169888 | EDE | North Branson 345 kV New Substation | 2024 ITP | 39,574,341 | 0 |
| 220903 | 5/1/2025 | 169890 | EDE | North Branson 345/161 kV Transformer Ckt 1 (161 kV) | 2024 ITP | 22,295,000 | 0 |
| 220903 | 5/1/2025 | 169891 | EDE | North Branson 345/161 kV Transformer Ckt 1 (345 kV) | 2024 ITP | 2,328,339 | 0 |
| 220903 | 5/1/2025 | 159430 | EDE | Aurora - Reeds Spring 161 kV Ckt 1 Rebuild | 2024 ITP | 37,387,772 | 0 |
| 220903 | 5/1/2025 | 159431 | EDE | Aurora 161 kV Ckt 1 Terminal Upgrade | 2024 ITP | 391,002 | 0 |
| 220903 | 5/1/2025 | 159432 | EDE | Reeds Spring 161 kV Ckt 1 Terminal Upgrade #1 | 2024 ITP | 126,095 | 0 |
| 220903 | 5/1/2025 | 170356 | EDE | Ozark Dam - Forsythe North 161 kV Ckt 1 Voltage Conversion | 2024 ITP | 6,062,305 | 0 |
| 220903 | 5/1/2025 | 170357 | EDE | Forsythe North 161 kV Ckt 1 Terminal Upgrade | 2024 ITP | 17,072,776 | 0 |
| 220903 | 5/1/2025 | 170358 | EDE | Forsythe North - Ozark South 161 kV Ckt 1 Voltage Conversion | 2024 ITP | 39,033,209 | 0 |
| 220903 | 5/1/2025 | 170359 | EDE | Ozark Dam 161 kV Ckt 1 Terminal Upgrade #1 | 2024 ITP | 1,048,264 | 0 |
| 220903 | 5/1/2025 | 170360 | EDE | Ozark South 161 kV Ckt 1 Terminal Upgrade | 2024 ITP | 1,994,954 | 0 |
| 220903 | 5/1/2025 | 170391 | EDE | Forsythe North 161 kV Ckt 1 Terminal Upgrade #1 | 2024 ITP | 841,998 | 0 |
| 220903 | 5/1/2025 | 170133 | EDE | Aurora H.T. - Monett 161 kV Ckt 1 Rebuild | 2024 ITP | 21,553,353 | 0 |
| 220903 | 5/1/2025 | 170134 | EDE | Aurora H.T. 161 kV Ckt 1 Terminal Upgrade | 2024 ITP | 538,096 | 0 |
| 220903 | 5/1/2025 | 170141 | EDE | Monett 161 kV Ckt 1 Terminal Upgrade #1 | 2024 ITP | 744,098 | 0 |
| 220904 | 6/19/2025 | 170517 | EREC | Dolton Manual – Marion Industrial 115 kV Ckt 1 Voltage Conversion | 2024 ITP | 0 | 0 |
| 220904 | 6/19/2025 | 170455 | EREC | Oldham - Lake County 115 kV Ckt 1 New Line | 2024 ITP | 9,185,000 | 0 |
| 220904 | 6/19/2025 | 170457 | EREC | Lake County 115 kV Ckt 1 Terminal Upgrade #2 | 2024 ITP | 750,000 | 0 |
| 220904 | 6/19/2025 | 170458 | EREC | Oldham 115 kV New Motor Operated Switch (MOS) Substation | 2024 ITP | 165,000 | 0 |
| 220904 | 6/19/2025 | 170459 | EREC | Oldham - Kingsbury County 115 kV Ckt 1 New Line | 2024 ITP | 7,321,000 | 0 |
| 220904 | 6/19/2025 | 170467 | EREC | Kingsbury County 115 kV New Substation | 2024 ITP | 7,303,000 | 0 |
| 220904 | 6/19/2025 | 170468 | EREC | Lake Preston - Kingsbury County 115 kV Ckt 1 New Line | 2024 ITP | 3,406,000 | 0 |
| 220904 | 6/19/2025 | 170470 | EREC | Lake Preston 115 kV New Motor Operated Switch (MOS) | 2024 ITP | 165,000 | 0 |
| 220904 | 6/19/2025 | 170474 | EREC | Lake Preston - DeSmet 115 kV Ckt 1 New Line | 2024 ITP | 6,353,000 | 0 |
| 220904 | 6/19/2025 | 170478 | EREC | DeSmet 115 kV New Motor Operated Switch (MOS) | 2024 ITP | 165,000 | 0 |
| 220904 | 6/19/2025 | 170479 | EREC | Manchester - DeSmet 115 kV Ckt 1 New Line | 2024 ITP | 5,718,000 | 0 |
| 220904 | 6/19/2025 | 170482 | EREC | Manchester 115 kV New Substation | 2024 ITP | 5,206,000 | 0 |
| 220904 | 6/19/2025 | 170483 | EREC | Carpenter 115 kV New Substation | 2024 ITP | 2,588,000 | 0 |
| 220904 | 6/19/2025 | 170484 | EREC | Manchester - Carpenter 115 kV Ckt 1 New Line | 2024 ITP | 13,326,000 | 0 |
| 220904 | 6/19/2025 | 170487 | EREC | Arlington - Kingsbury County 115 kV Ckt 1 New Line | 2024 ITP | 10,003,000 | 0 |
| 220904 | 6/19/2025 | 170488 | EREC | Arlington 115 kV Breakers | 2024 ITP | 3,846,000 | 0 |
| 220904 | 6/19/2025 | 170489 | EREC | Kingsbury County 115 kV Capacitor Bank | 2024 ITP | 855,000 | 0 |
| 220904 | 6/19/2025 | 170490 | EREC | Carpenter 230/115 kV Transformer Ckt 1 (115 kV) | 2024 ITP | 4,317,000 | 0 |
| 220904 | 6/19/2025 | 170491 | EREC | Carpenter 230/115 kV Transformer Ckt 1 (230 kV) | 2024 ITP | 3,333,000 | 0 |
| 220904 | 6/19/2025 | 159073 | EREC | North Marion - Montrose Manual 115 kV Ckt 1 Voltage Conversion | 2024 ITP | 0 | 0 |
| 220904 | 6/19/2025 | 159118 | EREC | Dolton Manual 115 kV New Switch | 2024 ITP | 152,000 | 0 |
| 220904 | 6/19/2025 | 159121 | EREC | Marion Industrial 115 kV New Substation | 2024 ITP | 4,025,000 | 0 |
| 220904 | 6/19/2025 | 159123 | EREC | Parker City 115 kV New Substation | 2024 ITP | 3,525,000 | 0 |
| 220904 | 6/19/2025 | 159144 | EREC | Canistota 69 kV Substation Voltage Conversion | 2024 ITP | 275,000 | 0 |
| 220904 | 6/19/2025 | 159162 | EREC | Canistota 115 kV Capacitor | 2024 ITP | 400,000 | 0 |
| 220904 | 6/19/2025 | 159166 | EREC | Dolton Manual - Canistota 115 kV Ckt 1 Voltage Conversion | 2024 ITP | 0 | 0 |
| 220904 | 6/19/2025 | 159176 | EREC | VT Hanlon 115 kV Ckt 1 Terminal Upgrade | 2024 ITP | 1,235,000 | 0 |
| 220904 | 6/19/2025 | 159181 | EREC | Marion - Parker City 115 kV Ckt 1 Rebuild | 2024 ITP | 2,480,000 | 0 |
| 220904 | 6/19/2025 | 159183 | EREC | Marion Industrial - Marion 115 kV Ckt 1 Voltage Conversion | 2024 ITP | 0 | 0 |
| 220904 | 6/19/2025 | 159185 | EREC | Marion Industrial 115 kV Capacitor Bank | 2024 ITP | 565,000 | 0 |
| 220904 | 6/19/2025 | 159192 | EREC | Montrose Manual - VT Hanlon 115 kV Ckt 1 | 2024 ITP | 0 | 0 |
| 220904 | 6/19/2025 | 159195 | EREC | North Marion - Canistota 115 kV Ckt 1 Voltage Conversion | 2024 ITP | 0 | 0 |
| 220904 | 6/19/2025 | 159212 | EREC | Turkey Ridge Manual 115 kV New Switch | 2024 ITP | 152,000 | 0 |
| 220904 | 6/19/2025 | 159214 | EREC | Turkey Ridge Manual - Turner County 115 kV Ckt 1 New Line | 2024 ITP | 5,880,000 | 0 |
| 220904 | 6/19/2025 | 159216 | EREC | Turkey Ridge Manual - Hutchinson County 115 kV Ckt 1 New Line | 2024 ITP | 3,280,000 | 0 |
| 220904 | 6/19/2025 | 159220 | EREC | Parker City - Turner County 115 kV Ckt 1 New Line | 2024 ITP | 4,675,000 | 0 |
| 220904 | 6/19/2025 | 159229 | EREC | Turner County 115 kV New Substation | 2024 ITP | 5,575,000 | 0 |
| 220904 | 6/19/2025 | 159230 | EREC | Hutchinson County 115 kV New Substation | 2024 ITP | 4,825,000 | 0 |
| 220904 | 6/19/2025 | 169675 | EREC | Mount Vernon - Mitchell 69 kV Ckt 1 New Line | 2024 ITP | 2,472,000 | 0 |
| 220904 | 6/19/2025 | 169676 | EREC | Mitchell 69 kV New Substation | 2024 ITP | 3,947,000 | 0 |
| 220904 | 6/19/2025 | 169677 | EREC | Letcher - Plano 115 kV Ckt 1 New Line | 2024 ITP | 2,795,000 | 0 |
| 220904 | 6/19/2025 | 169678 | EREC | Letcher 115 kV Ckt 1 Terminal Upgrade | 2024 ITP | 1,936,000 | 0 |
| 220904 | 6/19/2025 | 169686 | EREC | Mitchell - Hanson County 69 kV Ckt 1 New Line | 2024 ITP | 4,993,000 | 0 |
| 220904 | 6/19/2025 | 169692 | EREC | Plano - Hanson County 115 kV Ckt 1 New Line | 2024 ITP | 8,017,000 | 0 |
| 220904 | 6/19/2025 | 169694 | EREC | Plano 115 kV New Motor Operated Switch (MOS) | 2024 ITP | 170,000 | 0 |
| 220904 | 6/19/2025 | 169695 | EREC | Letcher-Miner County 115 kV line extension | 2024 ITP | 1,527,000 | 0 |
| 220904 | 6/19/2025 | 169696 | EREC | Hanson County 115 kV New Substation | 2024 ITP | 2,046,000 | 0 |
| 220904 | 6/19/2025 | 169697 | EREC | Hanson County 69 kV New Substation | 2024 ITP | 5,441,000 | 0 |
| 220904 | 6/19/2025 | 169698 | EREC | Hanson County 69 kV Capacitor | 2024 ITP | 963,000 | 0 |
| 220904 | 6/19/2025 | 169699 | EREC | Hanson County 115/69 kV Transformer Ckt 1 (69 kV) | 2024 ITP | 3,075,000 | 0 |
| 220904 | 6/19/2025 | 169701 | EREC | Mount Vernon 69 kV Ckt 1 Terminal Upgrade 3 | 2024 ITP | 800,000 | 0 |
| 220904 | 6/19/2025 | 169933 | EREC | Madison - Lake County115 kV Ckt 1 New Line | 2024 ITP | 882,000 | 0 |
| 220904 | 6/19/2025 | 169935 | EREC | Madison SE - Lakeview 115 kV Ckt 1 Voltage Conversion | 2024 ITP | 360,000 | 0 |
| 220904 | 6/19/2025 | 169937 | EREC | Lake County - Howard 115 kV Ckt 1 New Line | 2024 ITP | 8,762,000 | 0 |
| 220904 | 6/19/2025 | 169940 | EREC | Howard - Howard City 115 kV Ckt 1 New Line | 2024 ITP | 1,962,000 | 0 |
| 220904 | 6/19/2025 | 169951 | EREC | Howard City - Miner County 115 kV Ckt 1 New Line | 2024 ITP | 4,742,000 | 0 |
| 220904 | 6/19/2025 | 169953 | EREC | Miner County 115 kV Ckt 1 Terminal Upgrade #2 | 2024 ITP | 760,000 | 0 |
| 220904 | 6/19/2025 | 169969 | EREC | VT Hanlon - Lakeview 115 kV Ckt 1 Voltage Conversion | 2024 ITP | 0 | 0 |
| 220904 | 6/19/2025 | 169970 | EREC | VT Hanlon 115 kV Ckt 1 Terminal Upgrade #1 | 2024 ITP | 1,235,000 | 0 |
| 220904 | 6/19/2025 | 169972 | EREC | Lake County 115 kV New Substation | 2024 ITP | 3,342,000 | 0 |
| 220904 | 6/19/2025 | 169973 | EREC | Madison SE 115 kV New Substation | 2024 ITP | 4,570,000 | 0 |
| 220904 | 6/19/2025 | 169974 | EREC | Howard 115 kV New Substation | 2024 ITP | 3,795,000 | 0 |
| 220904 | 6/19/2025 | 169975 | EREC | Howard City 115 kV New Motor Operated Switch (MOS) | 2024 ITP | 165,000 | 0 |
| 220904 | 6/19/2025 | 169977 | EREC | Lakeview 115 kV New Motor Operated Switch (MOS) | 2024 ITP | 165,000 | 0 |
| 220904 | 6/19/2025 | 169978 | EREC | Miner County 115 kV Ckt 1 Terminal Upgrade #1 | 2024 ITP | 760,000 | 0 |
| 220904 | 6/19/2025 | 169979 | EREC | Lake County 115/69 kV Transformer Ckt 1 (69 kV) | 2024 ITP | 3,344,000 | 0 |
| 220905 | 5/6/2025 | 169628 | BEPC | Logan - Leland Olds 345 kV Ckt 1 Voltage Conversion | 2024 ITP | 106,781,208 | 0 |
| 220905 | 5/6/2025 | 169632 | BEPC | Logan 345 kV Ckt 1 Terminal Addition | 2024 ITP | 3,829,372 | 0 |
| 220905 | 5/6/2025 | 169633 | BEPC | Leland Olds 345 kV Ckt 1 Terminal Upgrade | 2024 ITP | 7,973,419 | 0 |
| 220905 | 5/6/2025 | 169634 | BEPC | Logan - Crane Creek 345 kV Ckt 1 New Line | 2024 ITP | 145,299,619 | 0 |
| 220905 | 5/6/2025 | 169635 | BEPC | Logan 345 kV Ckt 1 Terminal Upgrade #1 | 2024 ITP | 3,846,328 | 0 |
| 220905 | 5/6/2025 | 169636 | BEPC | Crane Creek 345 kV Terminal Addition | 2024 ITP | 9,235,041 | 0 |
| 220905 | 5/6/2025 | 169637 | BEPC | Logan 345 kV New Substation | 2024 ITP | 15,293,911 | 0 |
| 220905 | 5/6/2025 | 169638 | BEPC | Logan 345/230 kV Transformer Ckt 1 (230 kV) | 2024 ITP | 16,427,065 | 0 |
| 220905 | 5/6/2025 | 169639 | BEPC | Logan 345 kV Terminal Equipment Ckt 1 (345 kV) | 2024 ITP | 2,419,081 | 0 |
| 220905 | 5/6/2025 | 169640 | BEPC | Logan 345/230 kV Transformer Ckt 2 (230 kV) #1 | 2024 ITP | 16,632,004 | 0 |
| 220905 | 5/6/2025 | 169641 | BEPC | Logan 345 kV Terminal Equipment Ckt 2 (345 kV) #1 | 2024 ITP | 2,419,081 | 0 |
| 220905 | 5/6/2025 | 170516 | BEPC | Logan 230 kV Terminal Upgrade | 2024 ITP | 6,588,882 | 0 |
| 220906 | 5/6/2025 | 169993 | BEPC | Patent Gate - Pioneer 345 kV Ckt 1 New Line | 2024 ITP | 128,061,208 | 0 |
| 220906 | 5/6/2025 | 169994 | BEPC | Patent Gate 345 kV Ckt 1 Terminal Upgrade | 2024 ITP | 589,680 | 0 |
| 220906 | 5/6/2025 | 169995 | BEPC | Pioneer 345 kV Ckt 1 Terminal Upgrade | 2024 ITP | 3,949,983 | 0 |
| 220907 | 5/6/2025 | 170131 | NPPD | Scottsbluff 115 kV Ckt 1 Terminal Upgrade | 2024 ITP | 385,333 | 0 |
| 220908 | 5/12/2025 | 112380 | WFEC | Mooreland 69 kV Breakers #2 | 2019 ITP | 0 | 685000 |
| 220909 | 5/12/2025 | 156288 | SPS | Dasco 115 kV Capacitor Bank | 2022 ITP | 0 | 5110649 |
| 220912 | 5/12/2025 | 170428 | EM | Nashua 345/161 kV Transformer Ckt 2 (161 kV) | 2024 ITP | 9,823,936 | 0 |
| 220912 | 5/12/2025 | 170429 | EM | Nashua 345/161 kV Terminal Equipment (345 kV) | 2024 ITP | 21,991,020 | 0 |
| 220913 | 5/12/2025 | 158572 | TSOK | Redbud - Mathewson 345 kV Ckt 1 New Line | 2023 ITP | 72,028,435 | 0 |
| 220914 | 5/13/2025 | 170417 | OGE | Muskogee 345 kV Ckt 1 Terminal Upgrade#1 | 2024 ITP | 4,186,340 | 0 |
| 220914 | 5/13/2025 | 170418 | OGE | Ft Smith 345 kV Ckt 2 Terminal Upgrade | 2024 ITP | 5,468,530 | 0 |
| 220914 | 5/13/2025 | 170419 | OGE | Ft Smith 500/345 kV Transformer Ckt 2 (345 kV) | 2024 ITP | 17,480,235 | 0 |
| 220914 | 5/13/2025 | 170420 | OGE | Highway 59 Tap - Tahlequah 345 kV Ckt 1 Voltage Conversion | 2024 ITP | 109,806,850 | 0 |
| 220914 | 5/13/2025 | 170421 | OGE | Muskogee - Tahlequah 161 kV Ckt 1 Rebuild | 2024 ITP | 9,432,689 | 0 |
| 220914 | 5/13/2025 | 170422 | OGE | Ft Smith - Muskogee 345 kV Ckt 2 New Line | 2024 ITP | 24,685,329 | 0 |
| 220914 | 5/13/2025 | 170546 | OGE | Muskogee - Tahlequah 345 kV Ckt 1 Rebuild | 2024 ITP | 80,871,234 | 0 |
| 220914 | 5/13/2025 | 170555 | OGE | Ft Smith 500/345 kV Transformer Ckt 2 Terminal Equipment (345 kV) | 2024 ITP | 3,467,800 | 0 |
| 220914 | 5/13/2025 | 170556 | OGE | Ft Smith 500/345 kV Transformer Ckt 2 Terminal Equipment (500 kV) | 2024 ITP | 30,718,812 | 0 |
| 220915 | 6/4/2025 | 170572 | LES | 84th and Bluff to North Bluff 115 kV Line |  | 7,361,808 | 0 |
| 220916 | 6/4/2025 | 170573 | LES | 8th and N - 2nd and N 115 kV Line Upgrade | SUS-040 | 531,015 | 0 |
| 220917 | 6/4/2025 | 170574 | LES | 8th and N - 17th and Holdrege 115 kV Upgrade | SUS-041 | 936,245 | 0 |
| 220918 | 6/5/2025 | 170633 | OGE | AES Cogen 161 kV Ckt 1 Terminal Upgrade | SPP-2024-AG2-AFS-3 | 1,566,485 | 0 |
| 220918 | 6/5/2025 | 170634 | OGE | Midwest - Franklin 138kV Line Reconductor | SPP-2024-AG2-AFS-3 | 2,000,000 | 0 |
| 220918 | 6/5/2025 | 170636 | OGE | Tarby 161 kV Ckt 1 Terminal Upgrade | SPP-2024-AG2-AFS-3 | 1,566,485 | 0 |
| 220919 | 7/8/2025 | 169762 | WAPA | Dawson County - Williston 230 kV Ckt 1 New Line | 2024 ITP | 237,958,276 | 0 |
| 220919 | 7/8/2025 | 169763 | WAPA | Dawson County 230 kV Ckt 1 Terminal Upgrade | 2024 ITP | 4,592,583 | 0 |
| 220919 | 7/8/2025 | 169764 | WAPA | Williston 230 kV Ckt 1 Terminal Upgrade | 2024 ITP | 3,999,147 | 0 |
| 220920 | 7/14/2025 | 170677 | GRDA | N.E.S - Oologah 138 kV Ckt 1 Rebuild (GRDA) | DPA-2024-March-1924 | 6,545,025 | 0 |
| 220920 | 7/14/2025 | 170679 | GRDA | Oologah 138 kV Terminal Upgrade | DPA-2024-March-1924 | 0 | 0 |
| 220920 | 7/14/2025 | 170682 | GRDA | Oologah 138 kV Ckt1 Terminal Upgrade #2 | DPA-2024-March-1924 | 0 | 0 |
| 220921 | 7/14/2025 | 170680 | AEP | N.E.S 138 kV Terminal Upgrade | DPA-2024-March-1924 | 0 | 0 |
| 220921 | 7/14/2025 | 170678 | AEP | N.E.S - Oologah 138 kV Ckt 1 Rebuild (AEP) | DPA-2024-March-1924 | 49,611 | 0 |
| 220922 | 7/15/2025 | 170638 | AEP | Bonanza 161 kV Ckt 1 Terminal Upgrades | SPP-2024-AG2-AFS-3 | 1,000,000 | 0 |
| 220923 | 7/21/2025 | 170518 | WAPA | Beresford 115 kV Ckt 1 Terminal Upgrade | 2024 ITP | 145,000 | 0 |
| 220923 | 7/21/2025 | 159234 | WAPA | Sioux Falls 115 kV Ckt 1 Terminal Upgrade | 2024 ITP | 145,000 | 0 |
| 220923 | 7/21/2025 | 159235 | WAPA | Beresford 115 kV - Sioux Falls 115 kV Line Tap | 2024 ITP | 2,904,148 | 0 |
| 220924 | 7/22/2025 | 170554 | EDE | STL439 5 - WWR477 161 kV Circuit 1 Rebuild | DPA-2024-October-2044 | 3,751,774 | 0 |
| 220925 | 8/7/2025 | 170157 | SPS | Medanos - Lynch 115 kV Ckt 1 New Line | 2024 ITP | 21,717,782 | 0 |
| 220926 | 8/8/2025 | 159471 | BEPC | Twelve Mile 345/115 kV Transformer Ckt 1 (115 kV) | 2024 ITP | 12,233,925 | 0 |
| 220926 | 8/8/2025 | 159472 | BEPC | Twelve Mile 345 kV Terminal Upgrade (345 kV) | 2024 ITP | 2,394,416 | 0 |
| 220926 | 8/8/2025 | 159473 | BEPC | Twelve Mile 345 kV New Substation | 2024 ITP | 16,533,147 | 0 |
| 220926 | 8/8/2025 | 159474 | BEPC | Springbrook - Twelve Mile 345 kV Ckt 1 New Line | 2024 ITP | 40,000,000 | 0 |
| 220926 | 8/8/2025 | 159475 | BEPC | Springbrook 345 kV Ckt 1 Terminal Upgrade | 2024 ITP | 4,780,649 | 0 |
| 220926 | 8/8/2025 | 159476 | BEPC | Twelve Mile 345 kV Ckt 1 Terminal Additon | 2024 ITP | 3,550,253 | 0 |
| 220928 | 8/13/2025 | 170003 | BEPC | Laramie River - New Underwood 345 kV Ckt 1 New Line (BEPC) | 2024 ITP | 582,799,370 | 0 |
| 220928 | 8/13/2025 | 170004 | BEPC | Laramie River 345 kV Ckt 1 Terminal Upgrade (BEPC) | 2024 ITP | 11,334,906 | 0 |
| 220929 | 8/13/2025 | 170005 | WAPA | New Underwood 2 345 kV Ckt 1 Terminal Upgrade | 2024 ITP | 9,747,839 | 0 |
| 220929 | 8/13/2025 | 170006 | WAPA | New Underwood 2 - Maurine 2 345 kV Ckt 1 New Line | 2024 ITP | 211,621,167 | 0 |
| 220929 | 8/13/2025 | 170012 | WAPA | New Underwood 2 345 kV Ckt 1 Terminal Upgrade #1 | 2024 ITP | 10,327,789 | 0 |
| 220929 | 8/13/2025 | 170016 | WAPA | New Underwood 2 345/230 kV Transformer Ckt 1 (230 kV) | 2024 ITP | 15,975,365 | 0 |
| 220929 | 8/13/2025 | 170017 | WAPA | New Underwood 2 345/230 kV Transformer Ckt 1 (345 kV) | 2024 ITP | 2,964,870 | 0 |
| 220929 | 8/13/2025 | 170014 | WAPA | New Underwood 2 345 kV New Substation | 2024 ITP | 15,827,515 | 0 |
| 220929 | 8/13/2025 | 170529 | WAPA | New Underwood 2 230 kV Ckt 1 Terminal Upgrade | 2024 ITP | 7,956,029 | 0 |
| 220930 | 8/19/2025 | 170229 | NPPD | Antelope - Holt County 345 kV Ckt 1 New Line | 2024 ITP | 69,160,208 | 0 |
| 220930 | 8/19/2025 | 170230 | NPPD | Holt County 345 kV Ckt 1 Terminal Upgrade | 2024 ITP | 5,350,202 | 0 |
| 220930 | 8/19/2025 | 170231 | NPPD | Antelope 345 kV Ckt 1 Terminal Upgrade | 2024 ITP | 3,934,947 | 0 |
| 220931 | 8/19/2025 | 170192 | NPPD | Ainsworth Wind - Bassett 115 kV Ckt 1 New Line | 2024 ITP | 31,006,000 | 0 |
| 220931 | 8/19/2025 | 170193 | NPPD | Ainsworth Wind 115 kV Ckt 1 Terminal Upgrade | 2024 ITP | 1,562,842 | 0 |
| 220931 | 8/19/2025 | 170194 | NPPD | Bassett 115 kV Ckt 1 Terminal Upgrade | 2024 ITP | 1,313,940 | 0 |
| 220932 | 8/19/2025 | 170519 | WAPA | Belfield 2 345/230 kV Transformer Ckt 1 (230 kV) | 2024 ITP | 10,000,000 | 0 |
| 220932 | 8/19/2025 | 170520 | WAPA | Belfield 230 kV Terminal Equipment | 2024 ITP | 100,000 | 0 |
| 220932 | 8/19/2025 | 170524 | WAPA | Maurine 230 kV Terminal Upgrade | 2024 ITP | 3,709,672 | 0 |
| 220932 | 8/19/2025 | 170525 | WAPA | Maurine 2 230 kV Terminal Upgrade | 2024 ITP | 2,774,719 | 0 |
| 220932 | 8/19/2025 | 170526 | WAPA | Maurine – Maurine 2 230 kV Ckt 1 New Line | 2024 ITP | 6,000,000 | 0 |
| 220932 | 8/19/2025 | 170527 | WAPA | Belfield 345 kV Terminal Equipment | 2024 ITP | 400,000 | 0 |
| 220932 | 8/19/2025 | 170015 | WAPA | Maurine 2 345 kV New Substation | 2024 ITP | 14,987,986 | 0 |
| 220932 | 8/19/2025 | 170018 | WAPA | Maurine 2 345/230 kV Transformer Ckt 1 (230 kV) | 2024 ITP | 15,681,376 | 0 |
| 220932 | 8/19/2025 | 170019 | WAPA | Maurine 2 345/230 kV Terminal Equipment | 2024 ITP | 2,703,604 | 0 |
| 220932 | 8/19/2025 | 170013 | WAPA | Maurine 2 345 kV Ckt 1 Terminal Upgrade #1 | 2024 ITP | 9,816,800 | 0 |
| 220932 | 8/19/2025 | 170000 | WAPA | Belfield 2 - Maurine 2 345 kV Ckt 1 New Line | 2024 ITP | 402,593,167 | 0 |
| 220932 | 8/19/2025 | 170001 | WAPA | Belfield 2 345 kV Ckt 1 Terminal Upgrade | 2024 ITP | 10,400,000 | 0 |
| 220932 | 8/19/2025 | 170002 | WAPA | Maurine 2 345 kV Ckt 1 Terminal Upgrade | 2024 ITP | 11,443,047 | 0 |
| 220934 | 9/9/2025 | 170393 | ITCGP | Elm Creek 345 kV Terminal Upgrade #3 | 2024 ITP | 8,466,971 | 0 |
| 220935 | 9/9/2025 | 170392 | NPPD | Elm Creek - Tobias 345 kV Ckt 1 New Line (NPPD) | 2024 ITP | 123,647,000 | 0 |
| 220935 | 9/9/2025 | 170380 | NPPD | Tobias 345 kV Ckt 1 Terminal Upgrade | 2024 ITP | 11,410,404 | 0 |
| 220936 | 9/9/2025 | 170558 | SPS | Bandit 765/345 kV Transformer Ckt 2 (345 kV) | 2024 ITP | 62,235,103 | 0 |
| 220936 | 9/9/2025 | 170559 | SPS | Tecovas 765/345 kV Transformer Ckt 2 (345 kV) | 2024 ITP | 62,235,103 | 0 |
| 220936 | 9/9/2025 | 170560 | SPS | Crossroads 765/345 kV Transformer Ckt 2 (345 kV) | 2024 ITP | 63,058,628 | 0 |
| 220936 | 9/9/2025 | 170561 | SPS | Bandit 765/345 kV Transformer Ckt 3 (345 kV) | 2024 ITP | 62,235,103 | 0 |
| 220936 | 9/9/2025 | 170562 | SPS | Tecovas 765/345 kV Transformer Ckt 3 (345 kV) | 2024 ITP | 62,235,103 | 0 |
| 220936 | 9/9/2025 | 170563 | SPS | Crossroads 765/345 kV Transformer Ckt 3 (345 kV) | 2024 ITP | 63,058,628 | 0 |
| 220936 | 9/9/2025 | 170564 | SPS | Crossroads 765 kV Reactor 1 | 2024 ITP | 37,244,896 | 0 |
| 220936 | 9/9/2025 | 170614 | SPS | Tecovas 765 kV Reactor 1 | 2024 ITP | 36,018,258 | 0 |
| 220936 | 9/9/2025 | 170615 | SPS | Bandit 765 kV Reactor 1 | 2024 ITP | 62,559,830 | 0 |
| 220936 | 9/9/2025 | 170616 | SPS | Crossroads 345 kV Substation Expansion | 2024 ITP | 15,254,945 | 0 |
| 220936 | 9/9/2025 | 170617 | SPS | Phantom 345 kV Substation Expansion | 2024 ITP | 20,430,706 | 0 |
| 220936 | 9/9/2025 | 170618 | SPS | Potter County 345 kV Substation Expansion #1 | 2024 ITP | 39,644,067 | 0 |
| 220936 | 9/9/2025 | 170051 | SPS | Tecovas - Crossroads 765 kV Ckt 1 New Line | 2024 ITP | 936,807,351 | 0 |
| 220936 | 9/9/2025 | 170091 | SPS | Bandit 765/345 kV Transformer Ckt 1 (345 kV) | 2024 ITP | 73,665,801 | 0 |
| 220936 | 9/9/2025 | 170096 | SPS | Tecovas 765/345 kV Transformer Ckt 1 (345 kV) | 2024 ITP | 73,665,801 | 0 |
| 220936 | 9/9/2025 | 170054 | SPS | Crossroads - Bandit 765 kV Ckt 1 New Line | 2024 ITP | 1,158,308,441 | 0 |
| 220936 | 9/9/2025 | 170067 | SPS | Crossroads 765 kV Reactor 2 | 2024 ITP | 69,919,661 | 0 |
| 220936 | 9/9/2025 | 170068 | SPS | Crossroads 765/345 kV Transformer Ckt 1 (345 kV) | 2024 ITP | 74,330,994 | 0 |
| 220936 | 9/9/2025 | 170078 | SPS | New 765 kV Substation at Crossroads | 2024 ITP | 163,882,248 | 0 |
| 220936 | 9/9/2025 | 170079 | SPS | New 765 kV Substation at Bandit | 2024 ITP | 156,031,580 | 0 |
| 220936 | 9/9/2025 | 170080 | SPS | New 765 kV Substation at Tecovas | 2024 ITP | 156,339,028 | 0 |
| 220936 | 9/9/2025 | 170742 | SPS | Bandit 345 kV Substation | 2024 ITP | 16,953,522 | 0 |
| 220936 | 9/9/2025 | 170743 | SPS | Tecovas 345 kV Substation | 2024 ITP | 16,362,464 | 0 |
| 220936 | 9/9/2025 | 170744 | SPS | Tecovas - Potter County 345 kV Ckt 1 New Line | 2024 ITP | 40,539,317 | 0 |
| 220936 | 9/9/2025 | 170745 | SPS | Tecovas - Potter County 345 kV Ckt 2 New Line | 2024 ITP | 40,539,317 | 0 |
| 220936 | 9/9/2025 | 170746 | SPS | Tecovas - Potter County 345 kV Ckt 3 New Line | 2024 ITP | 40,539,317 | 0 |
| 220936 | 9/9/2025 | 170747 | SPS | Bandit - Phantom 345 kV Ckt 1 New Line | 2024 ITP | 6,686,208 | 0 |
| 220936 | 9/9/2025 | 170748 | SPS | Bandit - Phantom 345 kV Ckt 2 New Line | 2024 ITP | 6,686,208 | 0 |
| 220936 | 9/9/2025 | 170749 | SPS | Bandit - Phantom 345 kV Ckt 3 New Line | 2024 ITP | 6,686,208 | 0 |
| 220937 | 9/18/2025 | 159336 | AEP | Buffalo Flats - Delaware 345 kV Ckt 1 New Line | 2024 ITP | 113,332,569 | 0 |
| 220937 | 9/18/2025 | 159337 | AEP | Delaware 345 kV Ckt 1 Terminal Upgrade | 2024 ITP | 11,201,155 | 0 |
| 220938 | 9/18/2025 | 159338 | EKC | Buffalo 345 kV Ckt 1 Terminal Upgrade | 2024 ITP | 5,698,178 | 0 |
| 220938 | 9/18/2025 | 170566 | EKC | Buffalo Flats - Delaware 345 kV Ckt 1 New Line (EKC) | 2024 ITP | 430,182,934 | 0 |
| 220939 | 9/23/2025 | 170009 | BEPC | Sidney 345 kV Ckt 1 Terminal Upgrade | 2024 ITP | 21,138,823 | 0 |
| 220940 | 9/23/2025 | 170008 | SEPC | Holcomb 345 kV Ckt 1 Terminal Upgrade | 2024 ITP | 14,343,048 | 0 |
| 220940 | 9/23/2025 | 170565 | SEPC | Holcomb - Sidney 345 kV Ckt 1 New Line (SEPC) | 2024 ITP | 500,116,820 | 0 |
| 220941 | 9/23/2025 | 170611 | TSGT | Holcomb - Sidney 345 kV Ckt 1 New Line (TSGT) | 2024 ITP | 515,000,000 | 0 |
| 220942 | 10/14/2025 | 170514 | WAPA | Forman 115 kV Terminal Upgrades | SUS-031 | 1,600,000 | 0 |
| 220943 | 10/14/2025 | 170492 | NIPCO | K335 - J6 Boyer 69kV Rebuild |  | 463,200 | 0 |
| 220943 | 10/14/2025 | 170495 | NIPCO | J17 69 kV New Substation (NIPCO) |  | 2,800,000 | 0 |
| 220943 | 10/14/2025 | 170496 | NIPCO | K338 - J17 69 kV Rebuild (NIPCO) |  | 1,101,600 | 0 |
| 220943 | 10/14/2025 | 170497 | NIPCO | Line - K335 - K338 69 kV Rebuild |  | 136,800 | 0 |
| 220943 | 10/14/2025 | 170620 | NIPCO | Denison - J6 Boyer 69kV Rebuild (NIPCO) |  | 2,073,600 | 0 |
| 220944 | 10/14/2025 | 170498 | CBPC | Rebuild the circuit from WAPA Denison to the new 69 kV tap station (NIPCO) |  | 1,580,500 | 0 |
| 220945 | 11/4/2025 | 170172 | SPS | Moore County - Xit 230 kV Ckt 1 New Line | 2024 ITP | 54,563,546 | 0 |
| 220945 | 11/4/2025 | 170173 | SPS | Moore County 230 kV Ckt 1 Terminal Upgrade | 2024 ITP | 10,632,427 | 0 |
| 220945 | 11/4/2025 | 170174 | SPS | Xit 230 kV Ckt 1 Terminal Upgrade | 2024 ITP | 6,641,650 | 0 |
| 220946 | 11/4/2025 | 170110 | AEP | Delaware 345 kV Ckt 1 Terminal Upgrade #2 | 2024 ITP | 15,031,457 | 0 |
| 220946 | 11/4/2025 | 170106 | AEP | Delaware - Monett 345 kV Ckt 1 New Line | 2024 ITP | 250,258,040 | 0 |
| 220947 | 11/4/2025 | 170107 | EDE | Monett 345 kV Ckt 1 Terminal Upgrade #6 | 2024 ITP | 3,610,024 | 0 |
| 220947 | 11/4/2025 | 170567 | EDE | Delaware - Monett 345 kV Ckt 1 New Line (EDE) | 2024 ITP | 163,676,482 | 0 |
| 220948 | 11/4/2025 | 169892 | EDE | Flint Creek - Brookline 345 kV Line Tap #2 | 2024 ITP | 4,625,515 | 0 |
| 220948 | 11/4/2025 | 169893 | EDE | Monett 345 kV New Substation | 2024 ITP | 28,286,872 | 0 |
| 220948 | 11/4/2025 | 169649 | EDE | North Branson - Monett 345 kV Ckt 1 New Line | 2024 ITP | 213,682,884 | 0 |
| 220948 | 11/4/2025 | 169650 | EDE | North Branson 345 kV Ckt 1 Terminal Upgrade #1 | 2024 ITP | 2,354,102 | 0 |
| 220948 | 11/4/2025 | 169651 | EDE | Monett Tap 345 kV Ckt 1 Terminal Upgrade | 2024 ITP | 3,615,480 | 0 |
| 220949 | 11/7/2025 | 170169 | SPS | San Sem Tap 115 kV New Substation | 2024 ITP | 9,321,219 | 0 |
| 220949 | 11/7/2025 | 170151 | SPS | San Andreas - Seminole 115 kV Line Tap | 2024 ITP | 7,746,098 | 0 |
| 220949 | 11/7/2025 | 170152 | SPS | San Sem Tap 115/69 kV Transformer Ckt 1 (69 kV) | 2024 ITP | 5,441,435 | 0 |
| 220949 | 11/7/2025 | 170153 | SPS | San Sem Tap 115/69 kV Transformer Ckt 1 (115 kV) | 2024 ITP | 2,186,308 | 0 |
| 220950 | 11/7/2025 | 170903 | AEP | Beckville - Bearcat Tap 69 kV Ckt 1 Rebuild | 2025 ITP | 4,575,000 | 0 |
| 220950 | 11/7/2025 | 170904 | AEP | Beckville 69 kV Ckt 1 Terminal Upgrade toward Bearcat Tap 69 kV | 2025 ITP | 1,000,000 | 0 |
| 220950 | 11/7/2025 | 170905 | AEP | Bearcat Tap 69 kV Ckt 1 Terminal Upgrade toward Beckville 69 kV | 2025 ITP | 1,000,000 | 0 |
| 220950 | 11/7/2025 | 170906 | AEP | Henderson REC - Poynter 69 kV Ckt 1 Rebuild | 2025 ITP | 19,259,000 | 0 |
| 220950 | 11/7/2025 | 170908 | AEP | Poynter 69 kV Ckt 1 Terminal Upgrade toward Henderson REC 69 kV | 2025 ITP | 1,000,000 | 0 |
| 220950 | 11/7/2025 | 170909 | AEP | Henderson REC - Bearcat Tap 69 kV Ckt 1 Rebuild | 2025 ITP | 26,647,000 | 0 |
| 220950 | 11/7/2025 | 170910 | AEP | Henderson REC 69 kV Ckt 1 Terminal Upgrade toward Bearcat Tap 69 kV | 2025 ITP | 2,500,000 | 0 |
| 220950 | 11/7/2025 | 170911 | AEP | Bearcat Tap 69 kV Ckt 1 Terminal Upgrade toward Henderson REC 69 kV | 2025 ITP | 0 | 0 |
| 220950 | 11/7/2025 | 170939 | AEP | Buford 69 kV Substation Upgrade | 2025 ITP | 500,000 | 0 |
| 220950 | 11/7/2025 | 170940 | AEP | DeKalb 69 kV Substation Upgrade | 2025 ITP | 500,000 | 0 |
| 220950 | 11/7/2025 | 170941 | AEP | Weyco - Valliant 138 kV Ckt 2 New Line | 2025 ITP | 10,545,400 | 0 |
| 220950 | 11/7/2025 | 170942 | AEP | Weyco 138 kV Substation Expansion or Rebuild | 2025 ITP | 9,921,543 | 0 |
| 220950 | 11/7/2025 | 170943 | AEP | Valliant 138 kV Substation Expansion or Rebuild | 2025 ITP | 3,893,656 | 0 |
| 220950 | 11/7/2025 | 170946 | AEP | Sardis - Enogex Wilburton Tap 138 kV Line Tap | 2025 ITP | 2,200,000 | 0 |
| 220950 | 11/7/2025 | 170947 | AEP | Yanush 138 kV New Substation | 2025 ITP | 9,800,000 | 0 |
| 220950 | 11/7/2025 | 170948 | AEP | Talihina - Yanush 138 kV Ckt 1 New Line | 2025 ITP | 35,884,000 | 0 |
| 220950 | 11/7/2025 | 170953 | AEP | Lindsay Water Flood - Choctaw 138 kV Ckt 1 New Line | 2025 ITP | 13,189,000 | 0 |
| 220950 | 11/7/2025 | 170954 | AEP | Lindsay Water Flood 138 kV Substation Expansion or Rebuild | 2025 ITP | 750,000 | 0 |
| 220950 | 11/7/2025 | 170955 | AEP | Choctaw 138 kV Substation Expansion or Rebuild | 2025 ITP | 9,500,000 | 0 |
| 220950 | 11/7/2025 | 170968 | AEP | Brandy Branch 138 kV Breaker Additions | 2025 ITP | 275,000 | 0 |
| 220950 | 11/7/2025 | 171012 | AEP | Line - Duncan - Duncan Tap - Duncan Eastside - Rush Springs Tap - Marlow 138 kV Rebuild | 2025 ITP | 30,500,000 | 0 |
| 220950 | 11/7/2025 | 171027 | AEP | Camp County POD 138 kV Substation Expansion or Rebuild | 2025 ITP | 10,000,000 | 0 |
| 220950 | 11/7/2025 | 170937 | AEP | Buford - DeKalb 69 kV Line Tap | 2025 ITP | 2,200,000 | 0 |
| 220950 | 11/7/2025 | 171094 | AEP | AEP-Diana East 138 kV Ckt 1 Terminal Upgrade toward URECC-Diana 138 kV | 2025 ITP | 1,000,000 | 0 |
| 220950 | 11/7/2025 | 171092 | AEP | AEP-Diana East 138 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220950 | 11/7/2025 | 171115 | AEP | Murvaul Tap Switch 138 kV Substation Expansion or Rebuild | 2025 ITP | 14,000,000 | 0 |
| 220950 | 11/7/2025 | 171111 | AEP | Tenaska Switch 345 kV Substation Expansion or Rebuild | 2025 ITP | 15,000,000 | 0 |
| 220950 | 11/7/2025 | 171120 | AEP | Turnertown 69 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220950 | 11/7/2025 | 171129 | AEP | AEP Baldwin 69 kV Ckt 1 Terminal Upgrade toward Baldwin 69 kV | 2025 ITP | 1,300,000 | 0 |
| 220950 | 11/7/2025 | 171139 | AEP | Liberty City - Kilgore 138 kV Ckt 1 New Line | 2025 ITP | 10,000,000 | 0 |
| 220950 | 11/7/2025 | 171140 | AEP | Liberty City 138 kV Substation Expansion or Rebuild | 2025 ITP | 13,000,000 | 0 |
| 220950 | 11/7/2025 | 171141 | AEP | Liberty City 138 kV Ckt 1 Terminal Upgrade toward Kilgore 138 kV | 2025 ITP | 5,050,000 | 0 |
| 220950 | 11/7/2025 | 171123 | AEP | Evenside 69 kV Substation Relay Upgrade | 2025 ITP | 0 | 0 |
| 220950 | 11/7/2025 | 171124 | AEP | North Laneville 69 kV Hard Tap Modification | 2025 ITP | 0 | 0 |
| 220950 | 11/7/2025 | 171125 | AEP | Fears Corner 69 kV Hard Tap Modification | 2025 ITP | 0 | 0 |
| 220950 | 11/7/2025 | 171156 | AEP | Perdue 138 kV Substation Upgrades | 2025 ITP | 500,000 | 0 |
| 220950 | 11/7/2025 | 171127 | AEP | AEP Baldwin 69 kV Substation Expansion or Rebuild | 2025 ITP | 3,610,000 | 0 |
| 220950 | 11/7/2025 | 171401 | AEP | Talihina 138/69 kV Ckt 1 Transformer | 2025 ITP | 17,000,000 | 0 |
| 220950 | 11/7/2025 | 171402 | AEP | Talihina West 69 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220950 | 11/7/2025 | 171403 | AEP | Talihina 138 kV Ckt 1 Terminal Upgrade toward Talihina West 69 kV | 2025 ITP | 0 | 0 |
| 220950 | 11/7/2025 | 171404 | AEP | Talihina West 69 kV Ckt 1 Terminal Upgrade toward Talihina 138 kV | 2025 ITP | 0 | 0 |
| 220950 | 11/7/2025 | 171405 | AEP | Adora - Pilgrims Pride 69 kV Ckt 1 New Line | 2025 ITP | 8,475,000 | 0 |
| 220950 | 11/7/2025 | 171406 | AEP | Adora 69 kV Substation Expansion or Rebuild | 2025 ITP | 14,700,000 | 0 |
| 220950 | 11/7/2025 | 171411 | AEP | Choctaw 138 kV Capacitor | 2025 ITP | 2,000,000 | 0 |
| 220950 | 11/7/2025 | 171964 | AEP | South Dekalb Switch Station 69 kV New Substation (AEP) | 2025 ITP | 5,237,500 | 0 |
| 220950 | 11/7/2025 | 171965 | AEP | Chapel Hill 138 kV Substation Expansion or Rebuild (AEP) | 2025 ITP | 4,410,000 | 0 |
| 220950 | 11/7/2025 | 171967 | AEP | Pilgrim's Rest REC (Williamson-Ziegler) 69 kV Substation Reconfiguration (AEP) | 2025 ITP | 5,485,000 | 0 |
| 220951 | 11/10/2025 | 171221 | EMW | KCI - Ferrelview 161 kV Ckt 1 Rebuild | 2025 ITP | 6,795,000 | 0 |
| 220951 | 11/10/2025 | 171222 | EMW | KCI 161 kV Ckt 1 Terminal Upgrade toward Ferrelview 161 kV | 2025 ITP | 3,100,000 | 0 |
| 220951 | 11/10/2025 | 171223 | EMW | Ferrelview 161 kV Ckt 1 Terminal Upgrade toward KCI 161 kV | 2025 ITP | 3,100,000 | 0 |
| 220951 | 11/10/2025 | 171224 | EMW | KCI - Platte City 161 kV Ckt 1 Rebuild | 2025 ITP | 10,125,000 | 0 |
| 220951 | 11/10/2025 | 171225 | EMW | KCI 161 kV Ckt 1 Terminal Upgrade toward Platte City 161 kV | 2025 ITP | 3,100,000 | 0 |
| 220951 | 11/10/2025 | 171226 | EMW | Platte City 161 kV Ckt 1 Terminal Upgrade toward KCI 161 kV | 2025 ITP | 3,100,000 | 0 |
| 220951 | 11/10/2025 | 171227 | EMW | Ferrelview - TWA 161 kV Ckt 1 Rebuild | 2025 ITP | 3,450,000 | 0 |
| 220951 | 11/10/2025 | 171228 | EMW | Ferrelview 161 kV Ckt 1 Terminal Upgrade toward TWA 161 kV | 2025 ITP | 3,100,000 | 0 |
| 220951 | 11/10/2025 | 171229 | EMW | TWA 161 kV Ckt 1 Terminal Upgrade toward Ferrelview 161 kV | 2025 ITP | 3,100,000 | 0 |
| 220951 | 11/10/2025 | 171230 | EMW | Roanridge - North Congress 161 kV Ckt 1 Rebuild | 2025 ITP | 2,730,000 | 0 |
| 220951 | 11/10/2025 | 171231 | EMW | Roanridge 161 kV Ckt 1 Terminal Upgrade toward North Congress 161 kV | 2025 ITP | 3,100,000 | 0 |
| 220951 | 11/10/2025 | 171232 | EMW | North Congress 161 kV Ckt 1 Terminal Upgrade toward Roanridge 161 kV | 2025 ITP | 3,100,000 | 0 |
| 220951 | 11/10/2025 | 171233 | EMW | Roanridge 161 kV Ckt 1 Terminal Upgrade toward First Creek 161 kV | 2025 ITP | 1,902,215 | 0 |
| 220951 | 11/10/2025 | 170916 | EMW | Maryville - Midway 161 kV Ckt 1 Rebuild | 2025 ITP | 23,579,742 | 0 |
| 220951 | 11/10/2025 | 170917 | EMW | Maryville 161 kV Ckt 1 Terminal Upgrade toward Midway 161 kV | 2025 ITP | 1,220,344 | 0 |
| 220951 | 11/10/2025 | 170918 | EMW | Midway 161 kV Ckt 1 Terminal Upgrade toward Maryville 161 kV | 2025 ITP | 185,723 | 0 |
| 220951 | 11/10/2025 | 170919 | EMW | St. Joseph - Avenue City Tap 161 kV Ckt 1 Rebuild | 2025 ITP | 4,771,300 | 0 |
| 220951 | 11/10/2025 | 170920 | EMW | St. Joseph 161 kV Ckt 1 Terminal Upgrade toward Avenue City Tap 161 kV | 2025 ITP | 1,655,440 | 0 |
| 220951 | 11/10/2025 | 170921 | EMW | Avenue City Tap 161 kV Ckt 1 Terminal Upgrade toward St. Joseph 161 kV | 2025 ITP | 62,975 | 0 |
| 220951 | 11/10/2025 | 170922 | EMW | Midway - Avenue City Tap 161 kV Ckt 1 Rebuild | 2025 ITP | 24,305,896 | 0 |
| 220951 | 11/10/2025 | 170923 | EMW | Midway 161 kV Ckt 1 Terminal Upgrade toward Avenue City Tap 161 kV | 2025 ITP | 213,774 | 0 |
| 220951 | 11/10/2025 | 170924 | EMW | Avenue City Tap 161 kV Ckt 1 Terminal Upgrade toward Midway 161 kV | 2025 ITP | 101,257 | 0 |
| 220951 | 11/10/2025 | 170851 | EMW | TWA - North Congress 161 kV Ckt 1 Rebuild | 2025 ITP | 6,088,666 | 0 |
| 220951 | 11/10/2025 | 170852 | EMW | TWA 161 kV Ckt 1 Terminal Upgrade toward North Congress 161 kV | 2025 ITP | 1,909,407 | 0 |
| 220951 | 11/10/2025 | 170853 | EMW | North Congress 161 kV Ckt 1 Terminal Upgrade toward TWA 161 kV | 2025 ITP | 205,322 | 0 |
| 220952 | 11/10/2025 | 171244 | EREC | Harrisburg - Dayton 115 kV Ckt 1 Rebuild | 2025 ITP | 1,200,000 | 0 |
| 220952 | 11/10/2025 | 171245 | EREC | Harrisburg 115 kV Ckt 1 Terminal Upgrade toward Dayton 115 kV | 2025 ITP | 275,000 | 0 |
| 220952 | 11/10/2025 | 171246 | EREC | Dayton 115 kV Ckt 1 Terminal Upgrade toward Harrisburg 115 kV | 2025 ITP | 560,000 | 0 |
| 220952 | 11/10/2025 | 171172 | EREC | Aurora Industrial - BMU East 115 kV Ckt 1 New Line (EREC) | 2025 ITP | 2,950,000 | 0 |
| 220952 | 11/10/2025 | 171173 | EREC | Aurora Industrial 115 kV Substation Expansion or Rebuild | 2025 ITP | 4,500,000 | 0 |
| 220952 | 11/10/2025 | 171175 | EREC | Aurora Industrial 115 kV Ckt 1 Terminal Upgrade toward BMU East 115 kV | 2025 ITP | 1,100,000 | 0 |
| 220952 | 11/10/2025 | 171279 | EREC | Virgil Fodness - La Valley 115 kV Ckt 1 Rebuild | 2025 ITP | 2,880,000 | 0 |
| 220952 | 11/10/2025 | 171280 | EREC | Virgil Fodness 115 kV Ckt 1 Terminal Upgrade toward La Valley 115 kV | 2025 ITP | 500,000 | 0 |
| 220952 | 11/10/2025 | 171281 | EREC | La Valley 115 kV Ckt 1 Terminal Upgrade toward Virgil Fodness 115 kV | 2025 ITP | 275,000 | 0 |
| 220952 | 11/10/2025 | 171282 | EREC | La Valley - Dayton 115 kV Ckt 1 Rebuild | 2025 ITP | 2,525,000 | 0 |
| 220952 | 11/10/2025 | 171283 | EREC | La Valley 115 kV Ckt 1 Terminal Upgrade toward Dayton 115 kV | 2025 ITP | 275,000 | 0 |
| 220952 | 11/10/2025 | 171284 | EREC | Dayton 115 kV Ckt 1 Terminal Upgrade toward La Valley 115 kV | 2025 ITP | 560,000 | 0 |
| 220953 | 11/10/2025 | 171191 | LEA | Plains Int 115/69 kV Ckt 2 Transformer | 2025 ITP | 3,558,015 | 0 |
| 220953 | 11/10/2025 | 171193 | LEA | Plains Int 69 kV Substation Expansion or Rebuild | 2025 ITP | 4,241,985 | 0 |
| 220953 | 11/10/2025 | 171195 | LEA | Plains Int 69 kV Ckt 2 Terminal Upgrade toward Plains Int 115 kV | 2025 ITP | 3,500,000 | 0 |
| 220954 | 11/10/2025 | 170935 | MIDW | Knoll - North Hays 115 kV Ckt 1 Rebuild (2) | 2025 ITP | 2,600,000 | 0 |
| 220954 | 11/10/2025 | 170912 | MIDW | Knoll 115 kV Ckt 1 Terminal Upgrade toward North Hays 115 kV | 2025 ITP | 1,725,000 | 0 |
| 220954 | 11/10/2025 | 170913 | MIDW | North Hays 115 kV Ckt 1 Terminal Upgrade toward Knoll 115 kV | 2025 ITP | 650,000 | 0 |
| 220954 | 11/10/2025 | 171389 | MIDW | Smoky Hills - Summit 230 kV Ckt 1 Rebuild (2) (MIDW) | 2025 ITP | 37,500,000 | 0 |
| 220954 | 11/10/2025 | 171390 | MIDW | Smoky Hills 230 kV Ckt 1 Terminal Upgrade toward Summit 230 kV | 2025 ITP | 500,000 | 0 |
| 220955 | 11/10/2025 | 171212 | GRDA | Pruek - Osage 69 kV Ckt 1 Rebuild | 2025 ITP | 7,582,000 | 0 |
| 220955 | 11/10/2025 | 171213 | GRDA | Pruek 69 kV Ckt 1 Terminal Upgrade toward Osage 69 kV | 2025 ITP | 1,300,000 | 0 |
| 220955 | 11/10/2025 | 171214 | GRDA | Osage 69 kV Ckt 1 Terminal Upgrade toward Pruek 69 kV | 2025 ITP | 1,300,000 | 0 |
| 220956 | 11/10/2025 | 171966 | MRES | Aurora Industrial - BMU East 115 kV Ckt 1 New Line (MRES) | 2025 ITP | 2,950,000 | 0 |
| 220956 | 11/10/2025 | 171176 | MRES | BMU East 115 kV Ckt 1 Terminal Upgrade toward Aurora Industrial 115 kV | 2025 ITP | 1,100,000 | 0 |
| 220956 | 11/10/2025 | 171177 | MRES | Energize BMU Brookings East - BMU Brookings West 115 kV Ckt 1 | 2025 ITP | 3,300,000 | 0 |
| 220956 | 11/10/2025 | 171174 | MRES | BMU East 115 kV New Substation | 2025 ITP | 6,200,000 | 0 |
| 220958 | 11/10/2025 | 170834 | MWE | Lindahl 115 kV Ckt 1 Terminal Upgrade toward Simpson 115 kV | 2025 ITP | 0 | 0 |
| 220958 | 11/10/2025 | 170835 | MWE | Simpson 115 kV Ckt 1 Terminal Upgrade toward Lindahl 115 kV | 2025 ITP | 50,000 | 0 |
| 220959 | 11/10/2025 | 171201 | NPPD | Columbus Southeast - Columbus Hydro 115 kV Ckt 1 Rebuild | 2025 ITP | 3,600,000 | 0 |
| 220959 | 11/10/2025 | 171202 | NPPD | Columbus Southeast 115 kV Ckt 1 Terminal Upgrade toward Columbus Hydro 115 kV | 2025 ITP | 150,000 | 0 |
| 220959 | 11/10/2025 | 171203 | NPPD | Columbus Hydro 115 kV Ckt 1 Terminal Upgrade toward Columbus Southeast 115 kV | 2025 ITP | 100,000 | 0 |
| 220959 | 11/10/2025 | 171003 | NPPD | Axtell 115/345 kV Ckt 2 Transformer | 2025 ITP | 15,500,000 | 0 |
| 220959 | 11/10/2025 | 171006 | NPPD | Axtell 115 kV Ckt 2 Terminal Upgrade toward Axtell 345 kV | 2025 ITP | 16,400,000 | 0 |
| 220959 | 11/10/2025 | 171007 | NPPD | Axtell 345 kV Ckt 2 Terminal Upgrade toward Axtell 115 kV | 2025 ITP | 8,000,000 | 0 |
| 220960 | 11/10/2025 | 171168 | WAPA | Aurora 115 kV Substation Expansion or Rebuild | 2025 ITP | 11,000,000 | 0 |
| 220960 | 11/10/2025 | 171169 | WAPA | White 115 kV Substation Expansion or Rebuild | 2025 ITP | 11,000,000 | 0 |
| 220960 | 11/10/2025 | 171170 | WAPA | Aurora 115 kV Ckt 1 Terminal Upgrade toward White 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220960 | 11/10/2025 | 171171 | WAPA | White 115 kV Ckt 1 Terminal Upgrade toward Aurora 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220960 | 11/10/2025 | 171178 | WAPA | White 345/115 kV Ckt 2 Transformer | 2025 ITP | 10,000,000 | 0 |
| 220960 | 11/10/2025 | 171179 | WAPA | White 345 kV Substation Expansion or Rebuild | 2025 ITP | 2,000,000 | 0 |
| 220960 | 11/10/2025 | 171180 | WAPA | White 345 kV Ckt 2 Terminal Upgrade toward White 115 kV | 2025 ITP | 2,800,000 | 0 |
| 220960 | 11/10/2025 | 171181 | WAPA | White 115 kV Ckt 2 Terminal Upgrade toward White 345 kV | 2025 ITP | 2,800,000 | 0 |
| 220960 | 11/10/2025 | 171182 | WAPA | Aurora - White 115 kV Ckt 1 New Line (2) | 2025 ITP | 22,500,000 | 0 |
| 220960 | 11/10/2025 | 171236 | WAPA | Ft. Thompson 345/230 kV Ckt 3 Transformer | 2025 ITP | 18,450,000 | 0 |
| 220960 | 11/10/2025 | 171237 | WAPA | Ft. Thompson 345 kV Substation Expansion or Rebuild | 2025 ITP | 17,450,000 | 0 |
| 220960 | 11/10/2025 | 171238 | WAPA | Ft. Thompson 230 kV Substation Expansion or Rebuild | 2025 ITP | 4,200,000 | 0 |
| 220960 | 11/10/2025 | 171242 | WAPA | Ft. Thompson 345 kV Ckt 3 Terminal Upgrade toward Ft. Thompson 230 kV | 2025 ITP | 4,450,000 | 0 |
| 220960 | 11/10/2025 | 171243 | WAPA | Ft. Thompson 230 kV Ckt 3 Terminal Upgrade toward Ft. Thompson 345 kV | 2025 ITP | 4,450,000 | 0 |
| 220961 | 11/11/2025 | 170877 | BEPC | Patent Gate - Pioneer 345 kV Line Tap | 2025 ITP | 12,000,000 | 0 |
| 220961 | 11/11/2025 | 170878 | BEPC | Marley 345 kV New Substation | 2025 ITP | 24,188,071 | 0 |
| 220961 | 11/11/2025 | 170879 | BEPC | Marley 345/115 kV Ckt 1 Transformer | 2025 ITP | 15,886,035 | 0 |
| 220961 | 11/11/2025 | 170880 | BEPC | Marley 345 kV Ckt 1 Terminal Upgrade toward Marley 115 kV | 2025 ITP | 2,233,100 | 0 |
| 220961 | 11/11/2025 | 170881 | BEPC | Marley 115 kV Ckt 1 Terminal Upgrade toward Marley 345 kV | 2025 ITP | 404,917 | 0 |
| 220961 | 11/11/2025 | 170882 | BEPC | Marley 345/115 kV Ckt 2 Transformer | 2025 ITP | 16,198,065 | 0 |
| 220961 | 11/11/2025 | 170883 | BEPC | Marley 345 kV Ckt 2 Terminal Upgrade toward Marley 115 kV | 2025 ITP | 2,233,100 | 0 |
| 220961 | 11/11/2025 | 170884 | BEPC | Marley 115 kV Ckt 2 Terminal Upgrade toward Marley 345 kV | 2025 ITP | 404,917 | 0 |
| 220962 | 11/11/2025 | 170876 | EDE | Joplin West 7th 161 kV Circuit Switcher Brkr-16145 Replacement | 2025 ITP | 519,640 | 0 |
| 220963 | 11/11/2025 | 171008 | EKC | Altamont Tap - Katy 69 kV Ckt 1 Rebuild | 2025 ITP | 17,738,055 | 0 |
| 220963 | 11/11/2025 | 171009 | EKC | Altamont Tap 69 kV Ckt 1 Terminal Upgrade toward Katy 69 kV | 2025 ITP | 0 | 0 |
| 220963 | 11/11/2025 | 171010 | EKC | Katy 69 kV Ckt 1 Terminal Upgrade toward Altamont Tap 69 kV | 2025 ITP | 806,231 | 0 |
| 220963 | 11/11/2025 | 171011 | EKC | Katy 69 kV New Substation | 2025 ITP | 10,704,941 | 0 |
| 220963 | 11/11/2025 | 171256 | EKC | 166th Street 115 kV Ckt 1 Terminal Upgrade toward Jaggard 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220963 | 11/11/2025 | 171257 | EKC | Jaggard 115 kV Ckt 1 Terminal Upgrade toward 166th Street 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220963 | 11/11/2025 | 171258 | EKC | 166th Street 115 kV Ckt 1 Terminal Upgrade toward Jarbalo Jct. 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220963 | 11/11/2025 | 171259 | EKC | Jarbalo Jct. 115 kV Ckt 1 Terminal Upgrade toward 166th Street 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220963 | 11/11/2025 | 171260 | EKC | Jaggard 115 kV Ckt 1 Terminal Upgrade toward Pentagon 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220963 | 11/11/2025 | 171261 | EKC | Pentagon 115 kV Ckt 1 Terminal Upgrade toward Jaggard 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220963 | 11/11/2025 | 171262 | EKC | Pentagon 115 kV Ckt 1 Terminal Upgrade toward 53rd & Mund 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220963 | 11/11/2025 | 171263 | EKC | 53rd & Mund 115 kV Ckt 1 Terminal Upgrade toward Pentagon 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220963 | 11/11/2025 | 171391 | EKC | Summit 230 kV Ckt 1 Terminal Upgrade toward Smoky Hills 230 kV | 2025 ITP | 1,170,063 | 0 |
| 220963 | 11/11/2025 | 171392 | EKC | Smoky Hills - Summit 230 kV Ckt 1 Rebuild (2) (EKC) | 2025 ITP | 42,654,300 | 0 |
| 220964 | 11/11/2025 | 171412 | EM | Craig - Lenexa 161 kV Ckt 2 Rebuild (2) | 2025 ITP | 7,449,983 | 0 |
| 220964 | 11/11/2025 | 171234 | EM | First Creek 161 kV Ckt 1 Terminal Upgrade toward Roanridge 161 kV | 2025 ITP | 23,824 | 0 |
| 220964 | 11/11/2025 | 171235 | EM | Roanridge - First Creek 161 kV Ckt 1 Rebuild | 2025 ITP | 9,729,334 | 0 |
| 220964 | 11/11/2025 | 170868 | EM | Stilwell 161 kV Breaker R13-22 Replacement | 2025 ITP | 1,309,181 | 0 |
| 220964 | 11/11/2025 | 170869 | EM | Stilwell 161 kV Breaker R4-13 Replacement | 2025 ITP | 842,154 | 0 |
| 220964 | 11/11/2025 | 170870 | EM | Stilwell 161 kV Breaker R4-9 Replacement | 2025 ITP | 842,154 | 0 |
| 220964 | 11/11/2025 | 170871 | EM | Stilwell 161 kV Breaker R6-15 Replacement | 2025 ITP | 842,154 | 0 |
| 220964 | 11/11/2025 | 170872 | EM | Stilwell 161 kV Breaker R7-11 Replacement | 2025 ITP | 842,154 | 0 |
| 220964 | 11/11/2025 | 170873 | EM | Stilwell 161 kV Breaker R7-14 Replacement | 2025 ITP | 842,154 | 0 |
| 220964 | 11/11/2025 | 170874 | EM | Stilwell 161 kV Breaker R9-11 Replacement | 2025 ITP | 842,154 | 0 |
| 220964 | 11/11/2025 | 170875 | EM | Stilwell 161 kV Breaker R6-22 Replacement | 2025 ITP | 842,574 | 0 |
| 220964 | 11/11/2025 | 170914 | EM | Craig 161 kV Ckt 2 Terminal Upgrade toward Lenexa 161 kV | 2025 ITP | 1,775,287 | 0 |
| 220964 | 11/11/2025 | 170915 | EM | Lenexa 161 kV Ckt 2 Terminal Upgrade toward Craig 161 kV | 2025 ITP | 3,987,410 | 0 |
| 220965 | 11/11/2025 | 170831 | OGE | Trosper - Reno 138 kV Ckt 1 Rebuild | 2025 ITP | 3,278,000 | 0 |
| 220965 | 11/11/2025 | 170832 | OGE | Trosper 138 kV Ckt 1 Terminal Upgrade toward Reno 138 kV | 2025 ITP | 400,000 | 0 |
| 220965 | 11/11/2025 | 170833 | OGE | Reno 138 kV Ckt 1 Terminal Upgrade toward Trosper 138 kV | 2025 ITP | 2,500,000 | 0 |
| 220965 | 11/11/2025 | 170885 | OGE | Horseshoe Lake 138 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220965 | 11/11/2025 | 170886 | OGE | Midway 138 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220965 | 11/11/2025 | 170887 | OGE | Kelly Ave. - Remington Park Ckt 1 138 kV New Line | 2025 ITP | 4,100,000 | 0 |
| 220965 | 11/11/2025 | 170888 | OGE | Stonewall - Remington Park Ckt 1 138 kV New Line | 2025 ITP | 3,000,000 | 0 |
| 220965 | 11/11/2025 | 170889 | OGE | Remove Remington Tap - Remington 138 kV Ckt 1 | 2025 ITP | 0 | 0 |
| 220965 | 11/11/2025 | 170890 | OGE | Remington Park - NE 30th Ckt 1 138 kV Voltage Conversion | 2025 ITP | 7,000,000 | 0 |
| 220965 | 11/11/2025 | 170891 | OGE | Remington Park - Green Pastures Ckt 1 138 kV Voltage Conversion | 2025 ITP | 12,000,000 | 0 |
| 220965 | 11/11/2025 | 170892 | OGE | Remove Horseshoe Lake - Green Pastures 69 kV Ckt 1 | 2025 ITP | 0 | 0 |
| 220965 | 11/11/2025 | 170893 | OGE | Horseshoe Lake - Green Pastures Ckt 1 138 kV Voltage Conversion | 2025 ITP | 11,000,000 | 0 |
| 220965 | 11/11/2025 | 170894 | OGE | Remove NE 30th - Deep Fork Tap 69 kV Ckt 1 | 2025 ITP | 0 | 0 |
| 220965 | 11/11/2025 | 170895 | OGE | NE 30th - Reno Ckt 1 138 kV Voltage Conversion | 2025 ITP | 2,500,000 | 0 |
| 220965 | 11/11/2025 | 170896 | OGE | Remove Deep Fork Tap - Green Pastures 69 kV Ckt 1 | 2025 ITP | 0 | 0 |
| 220965 | 11/11/2025 | 170897 | OGE | Remove Deep Fork Tap - Deep Fork 69 kV Ckt 1 | 2025 ITP | 0 | 0 |
| 220965 | 11/11/2025 | 170898 | OGE | Remove Deep Fork Capacitor 69 kV Ckt 1 | 2025 ITP | 0 | 0 |
| 220965 | 11/11/2025 | 170899 | OGE | Remove Deep Fork Capacitor 69 kV Ckt 2 | 2025 ITP | 0 | 0 |
| 220965 | 11/11/2025 | 170900 | OGE | Remove Remington Tap 138 kV Substation | 2025 ITP | 0 | 0 |
| 220965 | 11/11/2025 | 170901 | OGE | Remove Deep Fork Tap 69 kV Substation | 2025 ITP | 0 | 0 |
| 220965 | 11/11/2025 | 170902 | OGE | Remove Deep Fork 69 kV Substation | 2025 ITP | 0 | 0 |
| 220965 | 11/11/2025 | 171059 | OGE | Muskogee - Ft. Smith 345 kV Line Tap | 2025 ITP | 500,000 | 0 |
| 220965 | 11/11/2025 | 171060 | OGE | Muskogee - Oktaha 345 kV Line Tap | 2025 ITP | 500,000 | 0 |
| 220965 | 11/11/2025 | 171061 | OGE | Muskogee 345 kV Ckt 2 Terminal Upgrade toward Keefton 345 kV | 2025 ITP | 10,000 | 0 |
| 220965 | 11/11/2025 | 171062 | OGE | Keefton 345 kV Ckt 2 Terminal Upgrade toward Muskogee 345 kV | 2025 ITP | 0 | 0 |
| 220965 | 11/11/2025 | 171063 | OGE | Pecan Creek - Keefton 345 kV Ckt 1 New Line | 2025 ITP | 37,332,865 | 0 |
| 220965 | 11/11/2025 | 171064 | OGE | Pecan Creek 345 kV Ckt 1 Terminal Upgrade toward Keefton 345 kV | 2025 ITP | 8,000,000 | 0 |
| 220965 | 11/11/2025 | 171065 | OGE | Keefton 345 kV Ckt 1 Terminal Upgrade toward Pecan Creek 345 kV | 2025 ITP | 0 | 0 |
| 220965 | 11/11/2025 | 171066 | OGE | Keefton 345 kV New Substation | 2025 ITP | 21,000,000 | 0 |
| 220965 | 11/11/2025 | 171157 | OGE | Lone Oak - Chisholm Creek 138 kV Ckt 1 New Line | 2025 ITP | 5,250,000 | 0 |
| 220965 | 11/11/2025 | 171158 | OGE | Lone Oak 138 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220965 | 11/11/2025 | 171159 | OGE | Chisholm Creek 138 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220965 | 11/11/2025 | 171160 | OGE | Lone Oak 138 kV Ckt 1 Terminal Upgrade toward Chisholm Creek 138 kV | 2025 ITP | 4,000,000 | 0 |
| 220965 | 11/11/2025 | 171161 | OGE | Chisholm Creek 138 kV Ckt 1 Terminal Upgrade toward Lone Oak 138 kV | 2025 ITP | 4,500,000 | 0 |
| 220965 | 11/11/2025 | 171002 | OGE | Pecan Creek 345 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220965 | 11/11/2025 | 170836 | OGE | Dunjee - Horseshoe Lake 138 kV Ckt 1 Rebuild | 2025 ITP | 7,948,700 | 0 |
| 220965 | 11/11/2025 | 170837 | OGE | Dunjee 138 kV Ckt 1 Terminal Upgrade toward Horseshoe Lake 138 kV | 2025 ITP | 10,000 | 0 |
| 220965 | 11/11/2025 | 170838 | OGE | Horseshoe Lake 138 kV Ckt 1 Terminal Upgrade toward Dunjee 138 kV | 2025 ITP | 650,000 | 0 |
| 220965 | 11/11/2025 | 170839 | OGE | Dunjee 138 kV Ckt 1 Terminal Upgrade toward Reno 138 kV | 2025 ITP | 10,000 | 0 |
| 220965 | 11/11/2025 | 170840 | OGE | Reno 138 kV Ckt 1 Terminal Upgrade toward Dunjee 138 kV | 2025 ITP | 650,000 | 0 |
| 220965 | 11/11/2025 | 170841 | OGE | Dunjee - Reno 138 kV Ckt 1 Rebuild | 2025 ITP | 7,393,200 | 0 |
| 220965 | 11/11/2025 | 170854 | OGE | NE 10th - Reno 138 kV Ckt 1 Rebuild | 2025 ITP | 4,992,000 | 0 |
| 220965 | 11/11/2025 | 170855 | OGE | NE 10th 138 kV Ckt 1 Terminal Upgrade toward Reno 138 kV | 2025 ITP | 400,000 | 0 |
| 220965 | 11/11/2025 | 170856 | OGE | Reno 138 kV Ckt 1 Terminal Upgrade toward NE 10th 138 kV | 2025 ITP | 3,000,000 | 0 |
| 220965 | 11/11/2025 | 170857 | OGE | Horseshoe Lake - Midway 138 kV Ckt 2 New Line | 2025 ITP | 7,800,000 | 0 |
| 220965 | 11/11/2025 | 170858 | OGE | Horseshoe Lake 138 kV Ckt 2 Terminal Upgrade toward Midway 138 kV | 2025 ITP | 2,000,000 | 0 |
| 220965 | 11/11/2025 | 170859 | OGE | Midway 138 kV Ckt 2 Terminal Upgrade toward Horseshoe Lake 138 kV | 2025 ITP | 4,500,000 | 0 |
| 220965 | 11/11/2025 | 170860 | OGE | Memorial 138 kV Ckt 1 Terminal Upgrade toward Bryant 138 kV | 2025 ITP | 500,000 | 0 |
| 220965 | 11/11/2025 | 170861 | OGE | Bryant 138 kV Ckt 1 Terminal Upgrade toward Memorial 138 kV | 2025 ITP | 10,000 | 0 |
| 220965 | 11/11/2025 | 170862 | OGE | Jones Tap 138 kV Ckt 1 Terminal Upgrade toward Horseshoe Lake 138 kV | 2025 ITP | 0 | 0 |
| 220965 | 11/11/2025 | 170863 | OGE | Horseshoe Lake 138 kV Ckt 1 Terminal Upgrade toward Jones Tap 138 kV | 2025 ITP | 1,000,000 | 0 |
| 220965 | 11/11/2025 | 170864 | OGE | Jones Tap 138 kV Ckt 1 Terminal Upgrade toward Acorn 138 kV | 2025 ITP | 500,000 | 0 |
| 220965 | 11/11/2025 | 170865 | OGE | Acorn 138 kV Ckt 1 Terminal Upgrade toward Jones Tap 138 kV | 2025 ITP | 10,000 | 0 |
| 220965 | 11/11/2025 | 170866 | OGE | NE 30th 69 kV to 138 kV Voltage Conversion | 2025 ITP | 9,000,000 | 0 |
| 220965 | 11/11/2025 | 170867 | OGE | Green Pastures 69 kV to 138 kV Voltage Conversion | 2025 ITP | 4,000,000 | 0 |
| 220965 | 11/11/2025 | 171960 | OGE | Horseshoe Lake 138 kV Ckt 1 Terminal Upgrade toward Green Pastures 138 kV | 2025 ITP | 4,000,000 | 0 |
| 220965 | 11/11/2025 | 171961 | OGE | Reno 138 kV Ckt 1 Terminal Upgrade toward NE 30th 138 kV | 2025 ITP | 2,500,000 | 0 |
| 220966 | 11/13/2025 | 171388 | OGE | Seminole 765 kV Line Reactor (2) | 2025 ITP | 6,565,000 | 0 |
| 220966 | 11/13/2025 | 171373 | OGE | Seminole 765 kV Ckt 1 Terminal Upgrade toward Anthem 765 kV | 2025 ITP | 18,000,000 | 0 |
| 220966 | 11/13/2025 | 171346 | OGE | Woodward 765 kV Ckt 1 Terminal Upgrade toward Crawfish Draw 765 kV | 2025 ITP | 18,000,000 | 0 |
| 220966 | 11/13/2025 | 171348 | OGE | Woodward 765 kV Line Reactor | 2025 ITP | 30,000,000 | 0 |
| 220966 | 11/13/2025 | 171349 | OGE | Woodward 765 kV New Substation | 2025 ITP | 80,000,000 | 0 |
| 220966 | 11/13/2025 | 171350 | OGE | Woodward 345 kV Reactor | 2025 ITP | 26,000,000 | 0 |
| 220966 | 11/13/2025 | 171351 | OGE | Woodward 345 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220966 | 11/13/2025 | 171352 | OGE | Woodward 765 kV Reactor | 2025 ITP | 30,000,000 | 0 |
| 220966 | 11/13/2025 | 171353 | OGE | Woodward 345/765 kV Ckt 1 Transformer | 2025 ITP | 61,500,000 | 0 |
| 220966 | 11/13/2025 | 171354 | OGE | Woodward 345 kV Ckt 1 Terminal Upgrade toward Woodward 765 kV | 2025 ITP | 5,000,000 | 0 |
| 220966 | 11/13/2025 | 171355 | OGE | Woodward 765 kV Ckt 1 Terminal Upgrade toward Woodward 345 kV | 2025 ITP | 18,000,000 | 0 |
| 220966 | 11/13/2025 | 171356 | OGE | Woodward 345/765 kV Ckt 2 Transformer | 2025 ITP | 61,500,000 | 0 |
| 220966 | 11/13/2025 | 171357 | OGE | Woodward 345 kV Ckt 2 Terminal Upgrade toward Woodward 765 kV | 2025 ITP | 5,000,000 | 0 |
| 220966 | 11/13/2025 | 171358 | OGE | Woodward 765 kV Ckt 2 Terminal Upgrade toward Woodward 345 kV | 2025 ITP | 18,000,000 | 0 |
| 220967 | 11/13/2025 | 171271 | AEP | Diana East 345/138 kV Ckt 1 Transformer | 2025 ITP | 10,800,000 | 0 |
| 220967 | 11/13/2025 | 171272 | AEP | Diana East 345 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220967 | 11/13/2025 | 171273 | AEP | Diana East 345 kV Ckt 1 Terminal Upgrade toward Diana 138 kV | 2025 ITP | 1,537,869 | 0 |
| 220967 | 11/13/2025 | 171274 | AEP | Diana East 138 kV Ckt 1 Terminal Upgrade toward Diana East 345 kV | 2025 ITP | 425,503 | 0 |
| 220967 | 11/13/2025 | 171275 | AEP | Southwest Shreveport 345/138 kV Ckt 1 Transformer | 2025 ITP | 10,800,000 | 0 |
| 220967 | 11/13/2025 | 171276 | AEP | Southwest Shreveport 345 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220967 | 11/13/2025 | 171277 | AEP | Southwest Shreveport 345 kV Ckt 1 Terminal Upgrade toward Southwest Shreveport 138 kV | 2025 ITP | 1,537,869 | 0 |
| 220967 | 11/13/2025 | 171278 | AEP | Southwest Shreveport 138 kV Ckt 1 Terminal Upgrade toward Southwest Shreveport 345 kV | 2025 ITP | 425,503 | 0 |
| 220967 | 11/13/2025 | 171375 | AEP | Anthem 765 kV New Substation | 2025 ITP | 377,000,000 | 0 |
| 220967 | 11/13/2025 | 171378 | AEP | Anthem 345 kV Substation Expansion or Rebuild | 2025 ITP | 50,000,000 | 0 |
| 220967 | 11/13/2025 | 171285 | AEP | Southwest Shreveport 345 kV STATCOM Ckt 1 | 2025 ITP | 47,000,000 | 0 |
| 220967 | 11/13/2025 | 171286 | AEP | Southwest Shreveport 345 kV STATCOM Ckt 2 | 2025 ITP | 47,000,000 | 0 |
| 220967 | 11/13/2025 | 171287 | AEP | Diana East 345 kV STATCOM Ckt 1 | 2025 ITP | 47,000,000 | 0 |
| 220967 | 11/13/2025 | 171288 | AEP | Diana East 345 kV STATCOM Ckt 2 | 2025 ITP | 47,000,000 | 0 |
| 220968 | 11/13/2025 | 171268 | BEPC | Roundup 345 kV Ckt 1 Terminal Upgrade toward Belfield 345 kV | 2025 ITP | 5,000,000 | 0 |
| 220968 | 11/13/2025 | 171266 | BEPC | Roundup 345 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220968 | 11/13/2025 | 171270 | BEPC | Roundup 345 kV Reactor | 2025 ITP | 2,520,000 | 0 |
| 220968 | 11/13/2025 | 171264 | BEPC | Belfield - Roundup 345 kV Ckt 1 New Line | 2025 ITP | 156,240,000 | 0 |
| 220971 | 11/13/2025 | 171265 | WAPA | Belfield 345 kV Substation Expansion or Rebuild | 2025 ITP | 15,000,000 | 0 |
| 220971 | 11/13/2025 | 171957 | WAPA | Belfield 2 345 kV Terminal Upgrade | 2025 ITP | 4,200,000 | 0 |
| 220971 | 11/13/2025 | 171958 | WAPA | Belfield 345 kV Terminal Upgrade | 2025 ITP | 4,200,000 | 0 |
| 220971 | 11/13/2025 | 171959 | WAPA | Belfield to Belfield 2 345 kV Ckt 1 New Line | 2025 ITP | 5,100,000 | 0 |
| 220971 | 11/13/2025 | 171267 | WAPA | Belfield 2 or Belfield 345 kV Ckt 1 Terminal Upgrade toward Roundup 345 kV | 2025 ITP | 5,200,000 | 0 |
| 220971 | 11/13/2025 | 171269 | WAPA | Belfield 2 or Belfield 345 kV Reactor | 2025 ITP | 2,520,000 | 0 |
| 220972 | 11/13/2025 | 171359 | SPS | Quahada 115 kV Capacitor | 2025 ITP | 2,880,000 | 0 |
| 220972 | 11/13/2025 | 171360 | SPS | Quahada 115 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220972 | 11/13/2025 | 171361 | SPS | Lynch Tap - Pearl 115 kV Ckt 1 Rebuild | 2025 ITP | 4,853,022 | 0 |
| 220972 | 11/13/2025 | 171362 | SPS | Lynch Tap 115 kV Ckt 1 Terminal Upgrade toward Pearl 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220972 | 11/13/2025 | 171363 | SPS | Pearl 115 kV Ckt 1 Terminal Upgrade toward Lynch Tap 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220972 | 11/13/2025 | 171364 | SPS | Twist 115 kV Capacitor (2) | 2025 ITP | 4,500,000 | 0 |
| 220972 | 11/13/2025 | 171365 | SPS | Twist 115 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220972 | 11/13/2025 | 171366 | SPS | Hobbs 230/345 kV Ckt 2 Transformer | 2025 ITP | 16,400,000 | 0 |
| 220972 | 11/13/2025 | 171367 | SPS | Hobbs 230 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220972 | 11/13/2025 | 171368 | SPS | Hobbs 345 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220972 | 11/13/2025 | 171369 | SPS | Hobbs 230 kV Ckt 2 Terminal Upgrade toward Hobbs 345 kV | 2025 ITP | 936,276 | 0 |
| 220972 | 11/13/2025 | 171370 | SPS | Hobbs 345 kV Ckt 2 Terminal Upgrade toward Hobbs 230 kV | 2025 ITP | 1,537,869 | 0 |
| 220972 | 11/13/2025 | 171394 | SPS | Kiowa 345/115 kV Ckt 2 Transformer | 2025 ITP | 9,700,000 | 0 |
| 220972 | 11/13/2025 | 171395 | SPS | Kiowa 345 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220972 | 11/13/2025 | 171396 | SPS | Kiowa 115 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220972 | 11/13/2025 | 171397 | SPS | Kiowa 345 kV Ckt 2 Terminal Upgrade toward Kiowa 115 kV | 2025 ITP | 1,537,869 | 0 |
| 220972 | 11/13/2025 | 171398 | SPS | Kiowa 115 kV Ckt 2 Terminal Upgrade toward Kiowa 345 kV | 2025 ITP | 385,333 | 0 |
| 220972 | 11/13/2025 | 171247 | SPS | Hobbs - Millen 115 kV Ckt 1 Rebuild | 2025 ITP | 10,150,000 | 0 |
| 220972 | 11/13/2025 | 171248 | SPS | Hobbs 115 kV Ckt 1 Terminal Upgrade toward Millen 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220972 | 11/13/2025 | 171249 | SPS | Millen 115 kV Ckt 1 Terminal Upgrade toward Hobbs 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220972 | 11/13/2025 | 171250 | SPS | Taylor S.W. Station - Hobbs N.E. 115 kV Ckt 1 Rebuild | 2025 ITP | 3,490,000 | 0 |
| 220972 | 11/13/2025 | 171251 | SPS | Taylor S.W. Station 115 kV Ckt 1 Terminal Upgrade toward Hobbs N.E. 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220972 | 11/13/2025 | 171252 | SPS | Hobbs N.E. 115 kV Ckt 1 Terminal Upgrade toward Taylor S.W. Station 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220972 | 11/13/2025 | 171253 | SPS | Millen - Hobbs N.E. 115 kV Ckt 1 Rebuild | 2025 ITP | 3,324,677 | 0 |
| 220972 | 11/13/2025 | 171254 | SPS | Millen 115 kV Ckt 1 Terminal Upgrade toward Hobbs N.E. 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220972 | 11/13/2025 | 171255 | SPS | Hobbs N.E. 115 kV Ckt 1 Terminal Upgrade toward Millen 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220972 | 11/13/2025 | 171209 | SPS | Lubbock South - Wolfforth 230 kV Ckt 1 Rebuild | 2025 ITP | 27,778,000 | 0 |
| 220972 | 11/13/2025 | 171210 | SPS | Lubbock South 230 kV Ckt 1 Terminal Upgrade toward Wolfforth 230 kV | 2025 ITP | 3,600,000 | 0 |
| 220972 | 11/13/2025 | 171211 | SPS | Wolfforth 230 kV Ckt 1 Terminal Upgrade toward Lubbock South 230 kV | 2025 ITP | 3,600,000 | 0 |
| 220972 | 11/13/2025 | 171194 | SPS | Plains Int 115 kV Ckt 2 Terminal Upgrade toward Plains Int 69 kV | 2025 ITP | 385,333 | 0 |
| 220972 | 11/13/2025 | 171192 | SPS | Plains Int 115 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220972 | 11/13/2025 | 171183 | SPS | Maddox - Pearl 115 kV Ckt 1 Rebuild (2) | 2025 ITP | 15,090,000 | 0 |
| 220972 | 11/13/2025 | 171184 | SPS | Maddox 115 kV Ckt 1 Terminal Upgrade toward Pearl 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220972 | 11/13/2025 | 171185 | SPS | Pearl 115 kV Ckt 1 Terminal Upgrade toward Maddox 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220972 | 11/13/2025 | 171186 | SPS | Lamb Co. 69/115 kV Ckt 4 Transformer | 2025 ITP | 4,725,000 | 0 |
| 220972 | 11/13/2025 | 171187 | SPS | Lamb Co. 69 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220972 | 11/13/2025 | 171188 | SPS | Lamb Co. 115 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220972 | 11/13/2025 | 171189 | SPS | Lamb Co. 69 kV Ckt 4 Terminal Upgrade toward Lamb Co. 115 kV | 2025 ITP | 248,675 | 0 |
| 220972 | 11/13/2025 | 171190 | SPS | Lamb Co. 115 kV Ckt 4 Terminal Upgrade toward Lamb Co. 69 kV | 2025 ITP | 385,333 | 0 |
| 220972 | 11/13/2025 | 170995 | SPS | Lea National 115 kV Substation Expansion or Rebuild | 2025 ITP | 24,399,994 | 0 |
| 220972 | 11/13/2025 | 170996 | SPS | Kiowa - Road Runner 345 kV Line Tap | 2025 ITP | 4,120,902 | 0 |
| 220972 | 11/13/2025 | 170997 | SPS | Sand Dunes 345 kV New Substation | 2025 ITP | 17,445,707 | 0 |
| 220972 | 11/13/2025 | 170998 | SPS | Sand Dunes 115/345 kV Ckt 1 Transformer | 2025 ITP | 17,651,761 | 0 |
| 220972 | 11/13/2025 | 170999 | SPS | Sand Dunes 115 kV Substation Expansion or Rebuild | 2025 ITP | 25,871,328 | 0 |
| 220972 | 11/13/2025 | 171000 | SPS | Sand Dunes 115 kV Ckt 1 Terminal Upgrade toward Sand Dunes 345 kV | 2025 ITP | 703,866 | 0 |
| 220972 | 11/13/2025 | 171001 | SPS | Sand Dunes 345 kV Ckt 1 Terminal Upgrade toward Sand Dunes 115 kV | 2025 ITP | 946,024 | 0 |
| 220972 | 11/13/2025 | 171051 | SPS | Hobbs - Kiowa 345 kV Line Tap | 2025 ITP | 2,977,711 | 0 |
| 220972 | 11/13/2025 | 171052 | SPS | Lea National 345 kV New Substation | 2025 ITP | 26,127,351 | 0 |
| 220972 | 11/13/2025 | 171053 | SPS | Lea National 115/345 kV Ckt 1 Transformer | 2025 ITP | 16,908,188 | 0 |
| 220972 | 11/13/2025 | 171054 | SPS | Lea National 115 kV Ckt 1 Terminal Upgrade toward Lea National 345 kV | 2025 ITP | 851,207 | 0 |
| 220972 | 11/13/2025 | 171055 | SPS | Lea National 345 kV Ckt 1 Terminal Upgrade toward Lea National 115 kV | 2025 ITP | 1,022,599 | 0 |
| 220972 | 11/13/2025 | 171056 | SPS | Lea National 115/345 kV Ckt 2 Transformer | 2025 ITP | 16,908,188 | 0 |
| 220972 | 11/13/2025 | 171057 | SPS | Lea National 115 kV Ckt 2 Terminal Upgrade toward Lea National 345 kV | 2025 ITP | 973,459 | 0 |
| 220972 | 11/13/2025 | 171058 | SPS | Lea National 345 kV Ckt 2 Terminal Upgrade toward Lea National 115 kV | 2025 ITP | 900,573 | 0 |
| 220972 | 11/13/2025 | 170842 | SPS | Whitten - Jal 115 kV Ckt 1 Rebuild | 2025 ITP | 5,740,739 | 0 |
| 220972 | 11/13/2025 | 170843 | SPS | Whitten 115 kV Ckt 1 Terminal Upgrade toward Jal 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220972 | 11/13/2025 | 170844 | SPS | Jal 115 kV Ckt 1 Terminal Upgrade toward Whitten 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220972 | 11/13/2025 | 170845 | SPS | Hobbs South - Switch 4J44 115 kV Ckt 1 Rebuild | 2025 ITP | 1,900,000 | 0 |
| 220972 | 11/13/2025 | 170846 | SPS | Hobbs South 115 kV Ckt 1 Terminal Upgrade toward Switch 4J44 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220972 | 11/13/2025 | 170847 | SPS | Switch 4J44 115 kV Ckt 1 Terminal Upgrade toward Hobbs South 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220972 | 11/13/2025 | 170848 | SPS | Switch 4J44 - Hobbs West S.W. Sta. 115 kV Ckt 1 Rebuild | 2025 ITP | 1,000,000 | 0 |
| 220972 | 11/13/2025 | 170849 | SPS | Switch 4J44 115 kV Ckt 1 Terminal Upgrade toward Hobbs West S.W. Sta. 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220972 | 11/13/2025 | 170850 | SPS | Hobbs West S.W. Sta. 115 kV Ckt 1 Terminal Upgrade toward Switch 4J44 115 kV | 2025 ITP | 2,100,000 | 0 |
| 220973 | 11/20/2025 | 170454 | BEPC | Logan 115 kV Ckt 1 Terminal Upgrade | 2024 ITP | 2,800,000 | 0 |
| 220973 | 11/20/2025 | 169835 | BEPC | Logan 230 kV Ckt 1 Terminal Upgrade | 2024 ITP | 4,100,000 | 0 |
| 220973 | 11/20/2025 | 172013 | BEPC | Magic City - Logan 230 kV Ckt 1 New Line (BEPC) | 2024 ITP | 9,900,000 | 0 |
| 220974 | 11/21/2025 | 171393 | ETEC | Energize Jarred Williamson - V.B. Shaw Tap 69 kV Ckt NO | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171429 | ETEC | J.G. Walker - Cathey 69 kV Ckt NO Close in Normally Open Circuit | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171430 | ETEC | J.G. Walker 69 kV Substation Expansion or Rebuild | 2025 ITP | 3,510,000 | 0 |
| 220974 | 11/21/2025 | 171431 | ETEC | Cathey 69 kV Substation Upgrades | 2025 ITP | 880,000 | 0 |
| 220974 | 11/21/2025 | 171432 | ETEC | J.G. Walker 69 kV Ckt NO Terminal Upgrade toward Cathey 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171433 | ETEC | Cathey 69 kV Ckt NO Terminal Upgrade toward J.G. Walker 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171434 | ETEC | Nichols 69 kV Capacitor | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171435 | ETEC | Nichols 69 kV Substation Expansion or Rebuild | 2025 ITP | 12,720,000 | 0 |
| 220974 | 11/21/2025 | 171436 | ETEC | Pilgrim's Rest REC (Williamson-Ziegler) 69 kV Substation Reconfiguration (ETEC) | 2025 ITP | 5,485,000 | 0 |
| 220974 | 11/21/2025 | 171437 | ETEC | Pilgrim's Rest REC (Williamson-Ziegler) 69 kV Ckt 1 Terminal Upgrade toward Cathey 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171438 | ETEC | Pilgrim's Rest REC (Williamson-Ziegler) 69 kV Ckt 1 Terminal Upgrade toward AEP-Alba 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171439 | ETEC | Pilgrim's Rest REC (Williamson-Ziegler) 69 kV Ckt 1 Terminal Upgrade toward AEP-Westwood 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171440 | ETEC | Nichols 69 kV Ckt 1 Terminal Upgrade toward EO Brown 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171441 | ETEC | Nichols 69 kV Ckt 1 Terminal Upgrade toward E Benton 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171442 | ETEC | Nichols 69 kV Ckt NO Terminal Upgrade toward Hill-Wilson 69 kV (2) | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171443 | ETEC | J.G. Walker 69 kV Ckt 1 Terminal Upgrade toward EO Brown 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171444 | ETEC | Cathey 69 kV Ckt 1 Terminal Upgrade toward Pilgrim's Rest REC (Williamson-Ziegler) 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171445 | ETEC | EO Brown 69 kV Substation Expansion or Rebuild | 2025 ITP | 4,050,000 | 0 |
| 220974 | 11/21/2025 | 171446 | ETEC | EO Brown 69 kV Ckt 1 Terminal Upgrade toward JG Walker 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171447 | ETEC | EO Brown 69 kV Ckt 1 Terminal Upgrade toward Nichols 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171448 | ETEC | E Benton 69 kV Substation Expansion or Rebuild | 2025 ITP | 3,460,000 | 0 |
| 220974 | 11/21/2025 | 171449 | ETEC | E Benton 69 kV Ckt 1 Terminal Upgrade toward Quitman 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171450 | ETEC | E Benton 69 kV Ckt 1 Terminal Upgrade toward Nichols 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171451 | ETEC | Quitman 69 kV Substation Upgrades | 2025 ITP | 880,000 | 0 |
| 220974 | 11/21/2025 | 171452 | ETEC | Quitman 69 kV Ckt 1 Terminal Upgrade toward E Benton 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170936 | ETEC | Ripley - Adora 69 kV Ckt NO Switch in Normally Open | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170938 | ETEC | South Dekalb Switch Station 69 kV New Substation | 2025 ITP | 5,237,500 | 0 |
| 220974 | 11/21/2025 | 171095 | ETEC | URECC-Diana 138 kV Ckt 1 Terminal Upgrade toward AEP-Diana East 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171096 | ETEC | Remove AEP-Diana East - Diana Switch 138 kV Ckt 1 | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171097 | ETEC | URECC-Diana - Noonday 138 kV Ckt 1 Retermination | 2025 ITP | 1,730,000 | 0 |
| 220974 | 11/21/2025 | 171098 | ETEC | Noonday 138 kV Substation Expansion or Rebuild | 2025 ITP | 5,790,000 | 0 |
| 220974 | 11/21/2025 | 171099 | ETEC | URECC-Diana 138 kV Ckt 1 Terminal Upgrade toward Noonday 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171100 | ETEC | Noonday 138 kV Ckt 1 Terminal Upgrade toward URECC-Diana 138 kV | 2025 ITP | 2,020,000 | 0 |
| 220974 | 11/21/2025 | 171101 | ETEC | URECC-Hallsville - Noonday 138 kV Ckt 1 Rebuild | 2025 ITP | 14,010,000 | 0 |
| 220974 | 11/21/2025 | 171102 | ETEC | URECC-Hallsville 138 kV Ckt 1 Terminal Upgrade toward Noonday 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171103 | ETEC | Noonday 138 kV Ckt 1 Terminal Upgrade toward URECC-Hallsville 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171104 | ETEC | Diana Switch 138 kV Retire Substation | 2025 ITP | 440,000 | 0 |
| 220974 | 11/21/2025 | 171105 | ETEC | Lake Murvaul - Tenaska Gateway 138 kV Ckt 1 New Line | 2025 ITP | 22,840,000 | 0 |
| 220974 | 11/21/2025 | 171106 | ETEC | Lake Murvaul 138 kV Substation Expansion or Rebuild | 2025 ITP | 1,650,000 | 0 |
| 220974 | 11/21/2025 | 171107 | ETEC | Lake Murvaul 138 kV Ckt 1 Terminal Upgrade toward Tenaska Gateway 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171108 | ETEC | Tenaska Gateway 138 kV Ckt 1 Terminal Upgrade toward Lake Murvaul 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171109 | ETEC | Tenaska Gateway 138 kV New Substation | 2025 ITP | 19,030,000 | 0 |
| 220974 | 11/21/2025 | 171110 | ETEC | Tenaska Gateway 138/345 kV Ckt 3 Transformer | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171093 | ETEC | URECC-Diana 138 kV Substation Expansion or Rebuild | 2025 ITP | 10,310,000 | 0 |
| 220974 | 11/21/2025 | 171121 | ETEC | Sawmill 69 kV Ckt 1 Terminal Upgrade toward Turnertown 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171122 | ETEC | Turnertown 69 kV Ckt 1 Terminal Upgrade toward Sawmill 69 kV | 2025 ITP | 1,300,000 | 0 |
| 220974 | 11/21/2025 | 171112 | ETEC | Tenaska Gateway 138 kV Ckt 3 Terminal Upgrade toward Tenaska Switch 345 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171114 | ETEC | Murvaul Tap Switch - Lake Murvaul 138 kV Ckt 1 Line Reconductor | 2025 ITP | 18,230,000 | 0 |
| 220974 | 11/21/2025 | 171117 | ETEC | Lake Murvaul 138 kV Ckt 1 Terminal Upgrade toward Murvaul Tap Switch 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171118 | ETEC | Sawmill - Turnertown 69 kV Ckt 1 New Line | 2025 ITP | 38,480,000 | 0 |
| 220974 | 11/21/2025 | 171119 | ETEC | Sawmill 69 kV Substation Expansion or Rebuild | 2025 ITP | 5,040,000 | 0 |
| 220974 | 11/21/2025 | 171142 | ETEC | Kilgore 138 kV Ckt 1 Terminal Upgrade toward Liberty City 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171143 | ETEC | Big Sandy - Mt. Eara Ckt 1 138 kV Voltage Conversion | 2025 ITP | 39,940,000 | 0 |
| 220974 | 11/21/2025 | 171144 | ETEC | Remove Kilgore - Mt. Eara Tap 69 kV Ckt 1 | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171146 | ETEC | Mt. Eara - Kilgore Ckt 1 138 kV New Line | 2025 ITP | 21,680,000 | 0 |
| 220974 | 11/21/2025 | 171147 | ETEC | Mt. Eara 138 kV Substation Voltage Conversion | 2025 ITP | 6,970,000 | 0 |
| 220974 | 11/21/2025 | 171149 | ETEC | Kilgore 138 kV New Substation | 2025 ITP | 16,410,000 | 0 |
| 220974 | 11/21/2025 | 171150 | ETEC | Kilgore 69/138 kV Ckt 1 Transformer | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171151 | ETEC | Kilgore 69 kV Substation Expansion or Rebuild | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171152 | ETEC | Kilgore 69 kV Ckt 1 Terminal Upgrade toward Kilgore 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171153 | ETEC | Kilgore 138 kV Ckt 1 Terminal Upgrade toward Kilgore 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171154 | ETEC | Big Sandy 138 kV Substation Voltage Conversion | 2025 ITP | 9,010,000 | 0 |
| 220974 | 11/21/2025 | 171130 | ETEC | ETEC/PHEC-Baldwin Switch 69 kV Ckt 1 Terminal Upgrade toward AEP Baldwin 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171131 | ETEC | Switch Cross Roads - Leigh 69 kV Ckt 1 to Normally Open | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171132 | ETEC | Stanley - Huxley 69 kV Ckt 1 New Line | 2025 ITP | 60,160,000 | 0 |
| 220974 | 11/21/2025 | 171133 | ETEC | Stanley 69 kV Substation Expansion or Rebuild | 2025 ITP | 4,220,000 | 0 |
| 220974 | 11/21/2025 | 171134 | ETEC | Huxley 69 kV Substation Expansion or Rebuild | 2025 ITP | 6,080,000 | 0 |
| 220974 | 11/21/2025 | 171135 | ETEC | Hurstown 69 kV Substation Reconfiguration | 2025 ITP | 4,220,000 | 0 |
| 220974 | 11/21/2025 | 171136 | ETEC | Center South 69 kV Substation Terminal Upgrade | 2025 ITP | 1,000,000 | 0 |
| 220974 | 11/21/2025 | 171137 | ETEC | Stanley 69 kV Ckt 1 Terminal Upgrade toward Huxley 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171138 | ETEC | Huxley 69 kV Ckt 1 Terminal Upgrade toward Stanley 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171126 | ETEC | ETEC/PHEC-Baldwin Switch - PHEC-Baldwin 69 kV Ckt 1 New Line | 2025 ITP | 8,120,000 | 0 |
| 220974 | 11/21/2025 | 171128 | ETEC | PHEC-Baldwin 69 kV Substation Expansion or Rebuild | 2025 ITP | 5,670,000 | 0 |
| 220974 | 11/21/2025 | 171068 | ETEC | Lake Hawkins - Piney Wood 138 kV Ckt 1 New Line | 2025 ITP | 23,600,000 | 0 |
| 220974 | 11/21/2025 | 171069 | ETEC | Lake Hawkins 138 kV Substation Expansion or Rebuild | 2025 ITP | 10,050,000 | 0 |
| 220974 | 11/21/2025 | 171070 | ETEC | Piney Wood Substation Voltage Conversion from 69 kV to 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171071 | ETEC | Lake Hawkins 138 kV Ckt 1 Terminal Upgrade toward Piney Wood 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171072 | ETEC | Piney Wood 138 kV Ckt 1 Terminal Upgrade toward Lake Hawkins 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171073 | ETEC | Little Mound - Gilmer Ckt 1 138 kV Voltage Conversion | 2025 ITP | 17,790,000 | 0 |
| 220974 | 11/21/2025 | 171074 | ETEC | Gilmer 69 kV Substation Voltage Conversion | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171075 | ETEC | Little Mound 138 kV Ckt 1 Terminal Upgrade toward Gilmer 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171076 | ETEC | Gilmer 138 kV Ckt 1 Terminal Upgrade toward Little Mound 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171077 | ETEC | Little Mound - Piney Wood Ckt 1 138 kV Voltage Conversion | 2025 ITP | 14,600,000 | 0 |
| 220974 | 11/21/2025 | 171078 | ETEC | Little Mound 138 kV Ckt 1 Terminal Upgrade toward Piney Wood 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171079 | ETEC | Piney Wood 138 kV Ckt 1 Terminal Upgrade toward Little Mound 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171080 | ETEC | Gilmer 138 kV New Substation | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171081 | ETEC | Piney Wood 138 kV New Substation | 2025 ITP | 8,120,000 | 0 |
| 220974 | 11/21/2025 | 171082 | ETEC | Remove Gilmer 69/138 kV Ckt 1 Transformer | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171083 | ETEC | Little Mound 138/69 kV Ckt 2 Transformer | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171084 | ETEC | Little Mound 138 kV New Substation | 2025 ITP | 15,750,000 | 0 |
| 220974 | 11/21/2025 | 171085 | ETEC | Little Mound 138 kV Ckt 2 Terminal Upgrade toward Little Mound 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171086 | ETEC | Little Mound 69 kV Ckt 2 Terminal Upgrade toward Little Mound 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171087 | ETEC | URECC-Gum Springs to URECC-Hallsville 138 kV Ckt 1 New Line | 2025 ITP | 12,090,000 | 0 |
| 220974 | 11/21/2025 | 171088 | ETEC | URECC-Gum Springs 138 kV Substation Expansion or Rebuild | 2025 ITP | 10,500,000 | 0 |
| 220974 | 11/21/2025 | 171089 | ETEC | URECC Hallsville 138 kV Substation Expansion or Rebuild | 2025 ITP | 2,970,000 | 0 |
| 220974 | 11/21/2025 | 171090 | ETEC | URECC-Gum Springs 138 kV Ckt 1 Terminal Upgrade toward URECC-Hallsville 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171091 | ETEC | URECC-Hallsville 138 kV Ckt 1 Terminal Upgrade toward URECC-Gum Springs 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171028 | ETEC | Ebenezer 69 kV to 138 kV Voltage Conversion | 2025 ITP | 8,370,000 | 0 |
| 220974 | 11/21/2025 | 171029 | ETEC | Camp County POD 138 kV Ckt 1 Terminal Upgrade toward Ebenezer 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171030 | ETEC | Ebenezer 138 kV Ckt 1 Terminal Upgrade toward Camp County POD 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171031 | ETEC | Cox - Pittsburg 69 kV Ckt 1 Reroute Line | 2025 ITP | 2,400,000 | 0 |
| 220974 | 11/21/2025 | 171032 | ETEC | Cox 69 kV Ckt 1 Terminal Upgrade toward Pittsburg 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171033 | ETEC | Pittsburg 69 kV Ckt 1 Terminal Upgrade toward Cox 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171034 | ETEC | Remove Ebenezer - Camp County Tap 69 kV Ckt 1 | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171035 | ETEC | Remove Ebenezer - Pittsburg 69 kV Ckt 1 | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171036 | ETEC | Ebenezer - Pittsburg Ckt 1 138 kV Voltage Conversion | 2025 ITP | 15,730,000 | 0 |
| 220974 | 11/21/2025 | 171038 | ETEC | Ebenezer 138 kV Ckt 1 Terminal Upgrade toward Pittsburg 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171039 | ETEC | Pittsburg 138 kV Ckt 1 Terminal Upgrade toward Ebenezer 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171040 | ETEC | Remove Gilmer - Pittsburg 69 kV Ckt NO | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171041 | ETEC | Gilmer - Pittsburg (Cox Tap) 138 kV Ckt 1 Voltage Conversion | 2025 ITP | 29,300,000 | 0 |
| 220974 | 11/21/2025 | 171042 | ETEC | Gilmer 138 kV Substation Expansion or Rebuild | 2025 ITP | 9,290,000 | 0 |
| 220974 | 11/21/2025 | 171043 | ETEC | Gilmer 138 kV Ckt 1 Terminal Upgrade toward Pittsburg 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171044 | ETEC | Pittsburg 138 kV Ckt 1 Terminal Upgrade toward Gilmer 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171046 | ETEC | Pittsburg 138 kV New Substation | 2025 ITP | 15,080,000 | 0 |
| 220974 | 11/21/2025 | 171047 | ETEC | Pittsburg 69 kV Upgrade Substation | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171048 | ETEC | Pittsburg 138/69 kV Ckt 1 Transformer | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171049 | ETEC | Pittsburg 138 kV Ckt 1 Terminal Upgrade toward Pittsburg 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 171050 | ETEC | Pittsburg 69 kV Ckt 1 Terminal Upgrade toward Pittsburg 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170969 | ETEC | Jefferson Metering Point 138 kV Substation Expansion or Rebuild | 2025 ITP | 560,000 | 0 |
| 220974 | 11/21/2025 | 170970 | ETEC | Lake O The Pines 138 kV Ckt 1 Terminal Upgrade toward Jefferson Metering Point 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170971 | ETEC | Nichols - Hill-Wilson 69 kV Ckt NO Close in Normally Open Circuit | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170972 | ETEC | Hill-Wilson 69 kV Substation Expansion or Rebuild | 2025 ITP | 130,000 | 0 |
| 220974 | 11/21/2025 | 170973 | ETEC | Nichols 69 kV Ckt NO Terminal Upgrade toward Hill-Wilson 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170974 | ETEC | Hill-Wilson 69 kV Ckt NO Terminal Upgrade toward Nichols 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170975 | ETEC | Hill-Wilson 69 kV Ckt 1 Terminal Upgrade toward D.O. Aldridge 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170976 | ETEC | D.O. Aldridge 69 kV Substation Expansion or Rebuild | 2025 ITP | 3,830,000 | 0 |
| 220974 | 11/21/2025 | 170977 | ETEC | D.O. Aldridge 69 kV Ckt 1 Terminal Upgrade toward Hill-Wilson 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170978 | ETEC | D.O. Aldridge 69 kV Ckt 1 Terminal Upgrade toward Scroggins 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170958 | ETEC | Darco - Nesbitt 138 kV Ckt 1 New Line | 2025 ITP | 24,430,000 | 0 |
| 220974 | 11/21/2025 | 170959 | ETEC | Darco 138 kV Substation Expansion or Rebuild | 2025 ITP | 840,000 | 0 |
| 220974 | 11/21/2025 | 170960 | ETEC | Nesbitt 69 kV to 138 kV Voltage Conversion | 2025 ITP | 11,660,000 | 0 |
| 220974 | 11/21/2025 | 170961 | ETEC | Darco 138 kV Ckt 1 Terminal Upgrade toward Nesbitt 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170962 | ETEC | Nesbitt 69 kV Ckt 1 Terminal Upgrade toward Darco 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170963 | ETEC | Remove Lake O The Pines - Nesbitt 69 kV Ckt 1 | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170964 | ETEC | Lake O The Pines - Nesbitt Ckt 1 138 kV Voltage Conversion | 2025 ITP | 17,580,000 | 0 |
| 220974 | 11/21/2025 | 170965 | ETEC | Lake O The Pines 138 kV Substation Expansion or Rebuild | 2025 ITP | 9,310,000 | 0 |
| 220974 | 11/21/2025 | 170966 | ETEC | Lake O The Pines 138 kV Ckt 1 Terminal Upgrade toward Nesbitt 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170967 | ETEC | Nesbitt 69 kV Ckt 1 Terminal Upgrade toward Lake O The Pines 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170951 | ETEC | Center - Center South 138 kV Line Tap | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170952 | ETEC | DETEC Center 138 kV New Substation | 2025 ITP | 11,480,000 | 0 |
| 220974 | 11/21/2025 | 170925 | ETEC | Ripley - Argo 69 kV Ckt 1 Rebuild | 2025 ITP | 17,360,000 | 0 |
| 220974 | 11/21/2025 | 170926 | ETEC | Ripley 69 kV Ckt 1 Terminal Upgrade toward Argo 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170927 | ETEC | Argo 69 kV Ckt 1 Terminal Upgrade toward Ripley 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170928 | ETEC | Ripley 69 kV Substation Expansion or Rebuild | 2025 ITP | 8,240,000 | 0 |
| 220974 | 11/21/2025 | 170929 | ETEC | Adora 69 kV New Substation | 2025 ITP | 10,150,000 | 0 |
| 220974 | 11/21/2025 | 170930 | ETEC | Ripley 69 kV Ckt NO Terminal Upgrade toward Adora 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170931 | ETEC | Adora 69 kV Ckt NO Terminal Upgrade toward Ripley 69 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170932 | ETEC | Argo 138 kV Ckt 1 Terminal Upgrade toward Liberty Hill 138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170933 | ETEC | Liberty Hill 138 kV Ckt 1 Terminal Upgrade toward Argo138 kV | 2025 ITP | 0 | 0 |
| 220974 | 11/21/2025 | 170934 | ETEC | Chapel Hill 138 kV Substation Expansion or Rebuild (ETEC) | 2025 ITP | 4,410,000 | 0 |
| 220975 | 11/25/2025 | 171413 | AEP | West Booneville - Boonville 69 kV Ckt 1 Rebuild Line | SPP-2025-AG1-AFS-3 | 3,250,000 | 0 |
| 220976 | 11/26/2025 | 171414 | SPS | BRU Substation - XTO Mahoney 230 kV Ckt 1 Rebuild | SPP-2025-AG1-AFS-3 | 500,000 | 0 |
| 220977 | 11/26/2025 | 171415 | WFEC | Cleo Corner - Cleo Springs Switch Station 69 kV Ckt 1 Rebuild | SPP-2025-AG1-AFS-3 | 1,750,000 | 0 |
| 220978 | 11/26/2025 | 171416 | OGE | Colony - Battlefield 161 kV Ckt 1 Rebuild | SPP-2025-AG1-AFS-3 | 1,700,000 | 0 |
| 220978 | 11/26/2025 | 171417 | OGE | Horseshoe Lake West - Horseshoe Lake East 69 kV Ckt Rebuild | SPP-2025-AG1-AFS-3 | 50,000 | 0 |
| 220978 | 11/26/2025 | 171419 | OGE | Tarby - Shady Point 161 kV Ckt 1 Terminal Upgrades | SPP-2025-AG1-AFS-3 | 1,500,000 | 0 |
| 220978 | 11/26/2025 | 171963 | OGE | Tarby - R.S. Kerr Dam 161 kV Ckt 1 Terminal Upgrades | SPP-2025-AG1-AFS-3 | 200,000 | 0 |
| 220979 | 12/3/2025 | 170065 | SPS | Beckham County - Potter 345 kV Ckt 1 New Line (SPS) | 2024 ITP | 197,351,541 | 0 |
| 220980 | 12/3/2025 | 171454 | TSOK | Beckham County - Potter 345 kV Ckt 1 New Line (TSOK) | 2024 ITP | 25,200,389 | 0 |
| 220984 | 1/21/2026 | 171968 | WFEC | Franklin SW 138 kV Terminal Upgrades | SPP-2025-AG1-AFS-3 | $100,000 | $0 |
| 220985 | 1/22/2026 | 159506 | BEPC | W Bank South 345 kV New Substation | 2024 ITP | $0 | $22,026,821 |
| 220985 | 1/22/2026 | 159507 | BEPC | W Bank South 345/115 kV Transformer Ckt 1 (115 kV) | 2024 ITP | $0 | $11,947,960 |
| 220985 | 1/22/2026 | 159508 | BEPC | W Bank South 345/115 kV Terminal Equipment Ckt 1 (345 kV) | 2024 ITP | $0 | $2,827,608 |
| 220986 | 1/30/2026 | 159460 | EKC | Edwardsville 161/115 kV Transformer Ckt 1 | 2024 ITP | $0 | $6,373,025 |
| 220987 | 1/30/2026 | 171418 | SWPA | R.S. Kerr Switchyard - Sunset Corner 161 kV Ckt 1 Rebuild | SPP-2025-AG1-AFS-3 | $350,000 | $0.00 |
| 749 Upgrades Issued |  |  |  |  | $14,117,803,170 |  | $5,795,649 |

Table 3: Upgrades Issued NTCs

### Upgrades/NTCs Withdrawn

Figure 7: Upgrades/NTCs Withdrawn

| NTC ID | NTC WITHDRAWN DATE |  | UID | OWNER | UPGRADE NAME | SOURCE STUDY | COST ESTIMATE OF WITHDRAWN UPGRADES |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 210671 |  | 5/3/2022 | 51170 | SPS | Amarillo South 230 kV Terminal Upgrades | 2015 ITPNT | $0 |
| 210668 |  | 4/27/2022 | 51623 | SPS | Tuco - Stanton 115 kV Terminal Upgrades | 2017 ITP10 | $0 |
| 210668 |  | 4/27/2022 | 61836 | SPS | Martin - Pantex North 115 kV Terminal Upgrades | 2017 ITP10 | $0 |
| 210668 |  | 4/27/2022 | 61837 | SPS | Pantex South - Highland Tap 115 kV Terminal Upgrades | 2017 ITP10 | $0 |
| 210672 |  | 5/5/2022 | 143161 | WAPA | Devil's Lake 115 kV Reactor | 2020 ITP | $3,171,019 |
| 210705 |  | 12/8/2022 | 122870 | SPS | Newhart - Potter 230 kV Terminal Upgrades | 2020 ITP | $0 |
| 210705 |  | 12/8/2022 | 143168 | SPS | Cargill - Deaf Smith #24 115 kV Ckt 1 Rebuild | 2020 ITP | $5,378,526 |
| 210705 |  | 12/8/2022 | 143169 | SPS | Parmer - Deaf Smith #24 115 kV Ckt 1 Rebuild | 2020 ITP | $999,423 |
| 210705 |  | 12/8/2022 | 143170 | SPS | Parmer - Deaf Smith #20 115 kV Ckt 1 Rebuild | 2020 ITP | $5,293,163 |
| 210707 |  | 12/8/2022 | 122733 | BEPC | Whitlock 230 kV Terminal Upgrades | 2021 ITP | $614,000 |
| 210666 |  | 4/27/2022 | 11027 | SPS | East Plant 115 kV Terminal Upgrade | SPP-2014-AG1-AFS-6 | $0 |
| 210667 |  | 4/27/2022 | 51409 | SPS | Potash Junction 230 kV Terminal Upgrade | SPP-2014-AG1-AFS-6 | $0 |
| 12 Upgrades Withdrawn |  |  |  |  |  |  | $15,456,131 |

Table 4: Upgrades Withdrawn

# In-Service Date Delays

Through the Project Cost Working Group (PCWG), stakeholders voiced concerns about SPP issued projects that were delayed past their need date and/or first reported in-service date. SPP staff and stakeholders worked together to develop a phased approach to fully address the issue and provide increased transparency on In-Service Date Delays.

The In-Service Date (ISD) Delay Report was developed and provided via a new section to the Quarterly Project Tracking (QPT) Report. The ISD Delay Report includes network upgrades that show an estimated in-service date at least one year past the identified in-service date at NTC acceptance also known as the First Reported In-Service Date. The ISD Report includes visuals of the delayed upgrades by severity, Transmission Owner, and cost.

1Q 2025 Quarterly Project Tracking Report Appendix 1&2 
2Q 2025 Quarterly Project Tracking Report Appendix 1&2 
3Q 2025 Quarterly Project Tracking Report Appendix 1&2
4Q 2025 Quarterly Project Tracking Report Appendix 1&2

# Study Overviews

### Transmission Service Studies

The Transmission Services department evaluates long-term requests for network integration and point-to-point transmission service through various processes, including the Aggregate Transmission Service Study (ATSS) process under Attachment Z1 of the tariff, performs screening studies through the Delivery Point Transfer (DPT) process under Attachment AR of the tariff and Delivery Point Addition (DPA) studies under Attachment AQ of the tariff. Below are links to studies posted during the reporting period.

### Completed Aggregate Facility Studies

| 2025-AG1-AFS-3 | Report Tables | Facility Log |
| --- | --- | --- |
| 2024-AG2-AFS-3 | Report Tables | Facility Log |

### Attachment AQ Requests

2025 Delivery Point Addition Studies

2024 Delivery Point Addition Studies

2023 Delivery Point Addition Studies

2022 Delivery Point Addition Studies

2021 Delivery Point Addition Studies

2020 Delivery Point Addition Studies

### Generator Interconnection Studies

SPP’s Generator Interconnection team creates stakeholder value by evaluating impacts of connecting new power sources to the existing transmission system. We facilitate agreements between generation producers and transmission owners while ensuring positive outcomes for the regional power grid.

Generator Interconnection Studies

### Transmission Planning (ITP)

SPP Board approved the 2025 ITP Assessment reports and portfolios on November 4, 2025.

2025 ITP Assessment Report

### Balanced Portfolio

SPP did not perform a Balanced Portfolio study during the reporting period.

### High-Priority Studies

SPP did not perform a High Priority study during the reporting period.

### Sponsored Upgrade Studies

| Study Name | ILTCRs Studied | Study Report Final | Board Approval |
| --- | --- | --- | --- |
| SUS-039 North Bluff - 84th & Bluff 115 kV Line |  | 9/20/2024 | 2/4/2025 |
| SUS-040 8th & N - 2nd & N (L1730) 115 kV Line Upgrade | Yes | 10/21/2024 | 2/4/2025 |
| SUS-041 8th & N - 17th & Holdrege (L1719) 115 kV Line Upgrade | Yes | 10/21/2024 | 2/4/2025 |

Table 5: 2025 Sponsored Upgrades

### Interregional Studies

SPP coordinates interregional transmission planning activities with neighboring Planning Regions as approved by FERC for Order 1000 compliance. SPP also coordinates interregional planning and operations activities across all applicable areas of the organization in accordance with SPP’s Joint Operating Agreements (JOAs) and other coordination arrangements that may not be required by Order 1000.

As required by the MISO-SPP JOA, the MISO-SPP Joint Planning Committee (JPC) mutually agreed to initiate a Coordinated System Plan (CSP) study in 2024 and in 2025 focused on screening analysis of the proposed transmission solutions being considered in the 2024 MISO-SPP CSP. The goal of the 2024 CSP study is to utilize a “blended” long-term planning model that represents the MISO and SPP regions in aggregate to identify immediately actionable transmission system enhancements that will improve reliability and resiliency in MISO and SPP. The “blended model” approach allows consideration of independent regional assumptions to be used as inputs on a model that includes the topology of both regions while considering the independent assumptions (e.g., generation and load growth) for each region. In 2025, the RTOs collectively evaluated transmission solutions that resolve reliability needs, provide transmission congestion savings, and enhance the transfer capability between the two systems. No transmission solutions will be formally approved for construction as a direct result of the 2024 MISO-SPP CSP, but the recommendations from the study will inform the next iteration of the MISO-SPP CSP, which is required by the MISO-SPP JOA to begin in 2026, as the RTOs pursue a robust quantitative and qualitative benefits analysis for the transmission solutions that warrant a closer review following the completion of the 2024 MISO-SPP CSP.

Building upon the momentum from 2024, in 2025 SPP and MISO continued implementation of the Joint Targeted Interconnection Queue (JTIQ) effort. The RTOs have jointly developed a “JTIQ Dashboard” site that provides detailed information about participation and commitments in the JTIQ process. The RTOs also enhanced the MISO-SPP JOA to consider interconnection requests submitted into each respective RTO’s expedited resource adequacy studies for inclusion in the JTIQ process. SPP and MISO transmission owners continued to make progress on constructing the five-project transmission portfolio that was approved by each RTO’s Board at the end of 2024, with projects expected to be placed in service between 2031 and 2034.

In mid-to-late 2025 SPP, Associated Electric Cooperative, Inc. (AECI), and Ameren (a transmission-owning member of MISO) partnered to jointly evaluate a proposed 345 kV transmission project that spans east to west across Missouri, referred to as the “Mid-MO” or “MoMENT” (Missouri Multi-Entity New Transmission) project. The segment of the project would add a 345 kV connection between the SPP (through Evergy Stilwell substation) and AECI (through Clinton/Golden Valley substation) systems. As of the end of 2025, the parties were continuing to evaluate potential cost sharing opportunities to support the applicable SPP stakeholder approvals for the construction of the SPP-to-AECI segment.

In addition to working with MISO and AECI, SPP participated in planning coordination with the Southeastern Regional Transmission Planning (SERTP) group in accordance with Addendum 4 to Attachment O of the SPP Tariff. In 2025, SPP stakeholders approved SPP Revision Request (RR) #741, which will add a new Addendum to SPP Tariff Attachment O to facilitate SPP’s participation in the FERC Order 1000-compliant interregional planning coordination process in the Western Interconnection in support of the SPP RTO Expansion project. This addition to Attachment O has been filed at FERC in docket ER26-1311.

# ITP20 Upgrades

ITP20 assessments were performed in 2010, 2013, and 2022. While the projects proposed by those studies are incorporated into the STEP Project List, they were not included in SPP’s project tracking effort as part of the Quarterly Tracking Report prior to Q1 2019. A list of active ITP20 projects will be maintained in the STEP Report and Quarterly Project Tracking Report lists going forward. The 2013 ITP20 projects are listed below in Table 6. The 2022 ITP20 projects are listed in Table 7.

| Project ID | Upgrade ID | Owner | Upgrade Name | Study | RTO Determined Need Date | Latest Cost Estimate |
| --- | --- | --- | --- | --- | --- | --- |
| 30401 | 50492 | TBD | Cass Co. - S.W. Omaha (aka S3454) 345 kV Ckt1 | 2013 ITP20 | 1/1/2033 | $33,126,800 |
| 30599 | 50769 | AEP | South Fayetteville 345/161 kV Transformer Ckt1 | 2013 ITP20 | 1/1/2033 | $12,600,000 |
| 30599 | 50770 | TBD | Chamber Springs - South Fayetteville 345 kV Ckt1 | 2013 ITP20 | 1/1/2033 | $21,295,800 |
| 30600 | 50771 | EMW | Maryville 345/161 kV Transformer Ckt1 | 2013 ITP20 | 1/1/2033 | $12,600,000 |
| 30602 | 50773 | TBD | Keystone - Red Willow 345 kV Ckt1 | 2013 ITP20 | 1/1/2033 | $130,141,000 |
| 30603 | 50774 | TBD | Tolk - Tuco 345 kV Ckt1 | 2013 ITP20 | 1/1/2033 | $75,718,400 |
| 30604 | 50775 | SEPC | Holcomb 345/115 kV Transformer Ckt2 | 2013 ITP20 | 1/1/2033 | $12,600,000 |
| 30605 | 50776 | TBD | Neosho - Wolf Creek 345 kV Ckt1 | 2013 ITP20 | 1/1/2033 | $117,126,900 |
| 30606 | 50777 | TBD | Clinton - Truman 161 kV Ckt1 Reconductor | 2013 ITP20 | 1/1/2033 | $15,701,325 |
| 30606 | 50778 | TBD | North Warsaw - Truman 161 kV Ckt1 Reconductor | 2013 ITP20 | 1/1/2033 | $1,082,850 |
| 30608 | 50779 | EKC | Auburn 345/115 kV Transformer Ckt2 | 2013 ITP20 | 1/1/2033 | $12,600,000 |
| 30608 | 50780 | TBD | Auburn - Swissvale 345 kV Ckt1 Voltage Conversion | 2013 ITP20 | 1/1/2033 | $20,112,700 |
| 30608 | 50781 | TBD | Auburn - Jeffrey EC 345 kV Ckt1 Voltage Conversion | 2013 ITP20 | 1/1/2033 | $35,493,000 |
| 30607 | 50782 | OGE | Muskogee/Pecan Creek 345 kV Terminal Upgrades | 2013 ITP20 | 1/1/2033 | $34,605,675 |
| 14 Total Projects |  |  |  |  |  | $534,804,450 |

Table 6. 2013 ITP20 projects

| Project ID | Upgrade ID | Owner | Upgrade Name | Study | RTO Determined Need Date | Latest Cost Estimate |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 93109 | 157811 | TBD | Ft. Smith - Thistle 345 kV CKT 1 | 2022 20 Year Assessment | 1/1/2042 | $461,160,280 |
| 93109 | 158510 | OGE | Ft. Smith 345 kV Terminal Equipment | 2022 20 Year Assessment | 1/1/2042 | $2,428,753 |
| 93109 | 158511 | SEPC | Thistle 345 kV Terminal Equipment | 2022 20 Year Assessment | 1/1/2042 | $2,428,753 |
| 93110 | 157812 | TBD | Flint Creek - Buffalo Flats 345 kV | 2022 20 Year Assessment | 1/1/2042 | $349,831,855 |
| 93110 | 158512 | EKC | Buffalo Flats 345 kV Terminal Equipment | 2022 20 Year Assessment | 1/1/2042 | $2,428,753 |
| 93110 | 158513 | AEP | Flint Creek 345 kV Terminal Equipment | 2022 20 Year Assessment | 1/1/2042 | $2,428,753 |
| 93111 | 157813 | OGE | Ft Smith - Sooner 345 kV CKT 1 | 2022 20 Year Assessment | 1/1/2042 | $290,563,847 |
| 93111 | 158514 | OGE | Ft. Smith 345 kV Terminal Equipment #2 | 2022 20 Year Assessment | 1/1/2042 | $2,428,753 |
| 93111 | 158515 | OGE | Sooner 345 kV Terminal Equipment | 2022 20 Year Assessment | 1/1/2042 | $2,428,753 |
| 93113 | 157905 | TBD | Huben - Lacygne 345 kV Ckt 1 New Line | 2022 20 Year Assessment | 1/1/2042 | $198,791,206 |
| 93113 | 158520 | AECI | Huben 345 kV Terminal Equipment | 2022 20 Year Assessment | 1/1/2042 | $2,428,753 |
| 93113 | 158521 | EM | Lacygne 345 kV Terminal Equipment | 2022 20 Year Assessment | 1/1/2042 | $2,428,753 |
| 93114 | 157824 | TBD | Riverside Station - Matthewson 345 kV (OGE) | 2022 20 Year Assessment | 1/1/2042 | $180,577,999 |
| 93114 | 158522 | OGE | Matthewson 345 kV Terminal Equipment | 2022 20 Year Assessment | 1/1/2042 | $2,428,753 |
| 93114 | 158523 | AEP | Riverside Station 345 kV Terminal Equipment | 2022 20 Year Assessment | 1/1/2042 | $2,428,753 |
| 93115 | 157825 | SPS | Potter County Interchange - Tolk Station 345 kV | 2022 20 Year Assessment | 1/1/2042 | $147,667,045 |
| 93115 | 158524 | SPS | Potter County Interchange 345 kV Terminal Equipment | 2022 20 Year Assessment | 1/1/2042 | $2,428,753 |
| 93115 | 158525 | SPS | Tolk Station 345 kV Terminal Equipment | 2022 20 Year Assessment | 1/1/2042 | $2,428,753 |
| 93116 | 157826 | MDU | Twin Brooks 3 to Ellendale 3 345 kV Tap | 2022 20 Year Assessment | 1/1/2042 | $16,467,621 |
| 93116 | 157827 | WAPA | Groton Tap - Groton 345 kV | 2022 20 Year Assessment | 1/1/2042 | $32,935,242 |
| 93116 | 157828 | WAPA | Broadland - Groton 345 kV | 2022 20 Year Assessment | 1/1/2042 | $32,935,242 |
| 93116 | 157873 | TBD | Twin Brooks 3 to Ellendale 3 345 kV Tap (OTP) | 2022 20 Year Assessment | 1/1/2042 | $16,467,621 |
| 93116 | 157875 | WAPA | Groton Tap 345 kV Sub | 2022 20 Year Assessment | 1/1/2042 | $0 |
| 93117 | 157829 | EM | Sibley - Nashua 345 kV | 2022 20 Year Assessment | 1/1/2042 | $43,969,609 |
| 93117 | 158526 | EM | Nashua 345 kV terminal Equipment | 2022 20 Year Assessment | 1/1/2042 | $2,428,753 |
| 93117 | 158527 | EM | Sibley 345 kV Terminal Equipment | 2022 20 Year Assessment | 1/1/2042 | $2,428,753 |
| 93120 | 157846 | TBD | Sibley - Hawthorn 345 kV CKT 2 | 2022 20 Year Assessment | 1/1/2042 | $25,368,257 |
| 93120 | 158530 | AEP | Hawthorn 345 kV Terminal Equipment #2 | 2022 20 Year Assessment | 1/1/2042 | $2,428,753 |
| 93120 | 158531 | EM | Sibley 345 kV Terminal Equipment #2 | 2022 20 Year Assessment | 1/1/2042 | $2,428,753 |
| 93121 | 157847 | WFEC | Anadarko 345/138 kV Transformer | 2022 20 Year Assessment | 1/1/2042 | $4,891,214 |
| 93121 | 157848 | TBD | Anadarko - Gracemont 345 kV | 2022 20 Year Assessment | 1/1/2042 | $5,619,548 |
| 93121 | 157849 | WFEC | Anadarko - Gracemont 345 kV (WFEC) | 2022 20 Year Assessment | 1/1/2042 | $5,619,547 |
| 93121 | 157850 | WFEC | Anadarko 345 kV Substation | 2022 20 Year Assessment | 1/1/2042 | $13,972,613 |
| 93121 | 122591 | OGE | Gracemont 345 kV Terminal Equipment | 2022 20 Year Assessment | 1/1/2042 | $0 |
| 93122 | 157851 | TBD | Cimarron - Norman Hill 345 kV CKT 2 | 2022 20 Year Assessment | 1/1/2042 | $20,616,250 |
| 93122 | 158532 | OGE | Cimarron 345 kV Terminal Equipment | 2022 20 Year Assessment | 1/1/2042 | $2,428,753 |
| 93122 | 158533 | OGE | Norman Hill 345 kV Terminal Equipment | 2022 20 Year Assessment | 1/1/2042 | $2,428,753 |
| 37 Total Projects |  |  |  |  |  | $1,891,172,550 |

Table 7: 2022 ITP20 Upgrades

# Appendix 1

(See accompanying list of STEP Projects.)